================================================================================

FILE: getAE.m

================================================================================

% =========================================================================

% getAE.m

% Alkaline Electrolyzer (AE) Stack Model

% Implements Equations 15-17 from:

% "PV/Hydrogen DC microgrid control using distributed economic MPC"

% Renewable Energy 222 (2024) 119871

%

% Inputs:

%   iae   : AE stack current (A) - scalar or vector

%

% Outputs:

%   vae   : AE stack voltage (V)

%   Pae   : AE stack power consumed (W)

%   NH2   : Hydrogen production rate (mol/s)

%   etaF  : Faraday (current) efficiency [-]

% =========================================================================

function [vae, Pae, NH2, etaF] = getAE(iae)

% =====================================================================

% AE STACK PARAMETERS  (Table 4 of reference paper)

% =====================================================================

Nae     = 30;           % Number of electrolyzer cells in series      [-]

Tae     = 25;           % Electrolyzer cell temperature                [deg C]

urev    = 1.229;        % Reversible cell voltage at 25C, 1atm        [V]

r1      = 7.33e-5;      % Ohmic resistance coefficient 1              [Ohm*m^2]

r2      = -1.11e-7;     % Ohmic resistance coefficient 2              [Ohm*m^2/degC]

te      = 0.185;        % Overvoltage coefficient                      [V]

t1      = 1.6e-2;       % Overvoltage parameter 1                     [m^2/A]

t2      = -1.302;       % Overvoltage parameter 2                     [m^2*degC/A]

t3      = 4.21e2;       % Overvoltage parameter 3                     [m^2*degC^2/A]

A       = 0.25;         % Electrolyzer cell active area               [m^2]

F       = 96485.3;      % Faraday constant                            [C/mol]

f1      = 2.5e4;        % Current efficiency parameter 1              [A^2/m^4]

f2      = 0.96;         % Current efficiency parameter 2              [-]

% Guard against zero or negative current

iae     = max(iae, 1e-6);

% =====================================================================

% STEP 1: Per-cell ohmic resistance term              [Part of Eq. 15]

% R_ohm = (r1 + r2*Tae) / A

% Units: [Ohm*m^2] / [m^2] = [Ohm]

% =====================================================================

R_ohm   = (r1 + r2 * Tae) / A;

% =====================================================================

% STEP 2: Per-cell overvoltage (Tafel) term          [Part of Eq. 15]

% arg = (t1 + t2/Tae + t3/Tae^2) / A * iae + 1

% V_over = te * ln(arg)

% =====================================================================

t_param = t1 + t2/Tae + t3/Tae^2;     % Combined temp-dependent term [m^2/A]

arg     = (t_param / A) .* iae + 1;    % Argument of logarithm        [-]

V_over  = te .* log(arg);              % Overvoltage per cell         [V]

% =====================================================================

% STEP 3: AE stack voltage                                    [Eq. 15]

% vae = Nae * { urev + R_ohm*iae + V_over }

% =====================================================================

vae     = Nae .* (urev + R_ohm .* iae + V_over);

% =====================================================================

% STEP 4: AE stack power consumed                             [Eq. 16]

% Pae = vae * iae

% =====================================================================

Pae     = vae .* iae;

% =====================================================================

% STEP 5: Faraday (current) efficiency                   [Below Eq. 17]

% etaF = [(iae/A)^2 / (f1 + (iae/A)^2)] * f2

% J = iae/A  [A/m^2]

% =====================================================================

J       = iae / A;                     % Current density              [A/m^2]

etaF    = (J.^2 ./ (f1 + J.^2)) * f2; % Current efficiency           [-]

% =====================================================================

% STEP 6: Hydrogen production rate                            [Eq. 17]

% NH2 = etaF * Nae * iae / (2*F)       [mol/s]

% =====================================================================

NH2     = etaF .* Nae .* iae ./ (2 * F);

end

================================================================================

FILE: getAEBuckConverter.m

================================================================================

% =========================================================================

% getAEBuckConverter.m

% Alkaline Electrolyzer Unidirectional Buck Converter State Equations

% Implements Equation 13 from Zhu et al., Renewable Energy 222 (2024) 119871

%

% Inputs:

%   iae   : AE inductor/AE current            (A)   [state x3]

%   Vdc   : DC bus voltage                    (V)   [state x5]

%   Sae   : Switch signal                     {0,1} [control input]

%

% Outputs:

%   diae  : d(iae)/dt                         (A/s)

%   ia    : Current drawn from DC bus         (A)

%   vae   : AE stack voltage                  (V)

%   Pae   : AE power consumed                 (W)

%   NH2   : Hydrogen production rate          (mol/s)

% =========================================================================

function [diae, ia, vae, Pae, NH2] = getAEBuckConverter(iae, Vdc, Sae)

% --- Converter parameter (Table 4, Zhu et al.) ---

Lae     = 01e-3;     % Buck converter inductance   [H]

% --- AE stack voltage: nonlinear coupling (Eq. 15) ---

% Clamp current positive; getAE handles its own internal guard

iae_safe            = max(iae, 0);

[vae, Pae, NH2, ~]  = getAE(iae_safe);

% --- State equation (Eq. 13) ---

% Lae * diae/dt = Sae * Vdc - vae(iae)

diae    = (Sae * Vdc - vae) / Lae;

% --- Current drawn from DC bus ---

% ia = Sae * iae

ia      = Sae * iae_safe;

end

================================================================================

FILE: getBattery.m

================================================================================

% =========================================================================

% getBattery.m

% Battery Energy Storage System (BESS) Model

%

% This function models a basic Battery using an internal resistance model

% and Coulomb counting for State of Charge (SOC) tracking.

%

% Inputs:

%   I_bat    : Battery current (A) - Positive for discharge, negative for charge

%   SOC      : Current State of Charge (fraction, 0 to 1)

%

% Outputs:

%   V_bat    : Battery terminal voltage (V)

%   dSOC     : Derivative of State of Charge (1/s)

%   P_bat    : Battery output power (W)

% =========================================================================

function [V_bat, dSOC, P_bat] = getBattery(I_bat, SOC)

% --- Battery Parameters ---

V_nom    = 240;            % Nominal voltage (V)

R_int    = 0.15;           % Internal resistance (Ohms)

C_Ah     = 100;            % Battery capacity in Ampere-hours (Ah)

C_As     = C_Ah * 3600;    % Battery capacity in Ampere-seconds (Coulombs)

% --- SOC Derivative ---

dSOC = -I_bat / C_As;

% --- Terminal Voltage Calculation ---

V_bat = V_nom - I_bat * R_int;

% --- Power Calculation ---

P_bat = V_bat * I_bat;

end

================================================================================

FILE: getBidirectionalConverter.m

================================================================================

% =========================================================================

% getBidirectionalConverter.m

% Bidirectional DC-DC Buck-Boost Converter for BESS

%

% Inputs:

%   ib     : Battery inductor current (A) [State x6]

%            (Positive = discharging, Negative = charging)

%   Vbat   : Battery terminal voltage (V)

%   Vdc    : DC bus voltage (V)

%   db     : Duty cycle of the lower switch (0 to 1)

%

% Outputs:

%   dib    : Derivative of battery inductor current (A/s)

%   ib_bus : Current injected into/drawn from the DC bus (A)

% =========================================================================

function [dib, ib_bus] = getBidirectionalConverter(ib, Vbat, Vdc, db)

Lb = 2e-3; % Battery converter inductance [H]

dib = (Vbat - (1 - db) * Vdc) / Lb;

ib_bus = (1 - db) * ib;

end

================================================================================

FILE: getDCBus.m

================================================================================

% =========================================================================

% getDCBus.m

% DC Bus Voltage Dynamics

% Implements Equation 10 from Zhu et al., Renewable Energy 222 (2024) 119871

%

% Inputs:

%   Vdc   : DC bus voltage                    (V)   [state x5]

%   is    : PV converter output current       (A)   [from getPVBoostConverter]

%   ip    : PEMFC converter output current    (A)   [from getPEMFCBoostConverter]

%   ia    : AE converter input current        (A)   [from getAEBuckConverter]

%   ib_bus: Battery converter output current  (A)   [from getBidirectionalConverter]

%   Pload : Load power demand                 (W)   [disturbance]

%

% Output:

%   dVdc  : d(Vdc)/dt                         (V/s)

%   il    : Load current                      (A)

% =========================================================================

function [dVdc, il] = getDCBus(Vdc, is, ip, ia, varargin)

if nargin == 5

ib_bus = 0;

Pload  = varargin{1};

else

ib_bus = varargin{1};

Pload  = varargin{2};

end

% --- DC bus parameter (Table 4, Zhu et al.) ---

Cdc     = 30e-3;    % DC bus capacitance          [F]

% --- Load current (constant power load model) ---

% il = Pload / Vdc

Vdc_safe    = max(Vdc, 1);      % Prevent division by zero

il          = Pload / Vdc_safe;

% --- DC bus state equation (Eq. 10) ---

% Cdc * dVdc/dt = is + ip + ib_bus - ia - il

dVdc    = (is + ip + ib_bus - ia - il) / Cdc;

end

%%

%

% ---

%

% **How these four files connect:**

% ```

% G, T, vpv ──► getPVBoostConverter ──► dvpv, diL, is, Ppv ──┐

%                                                               │

% iae, Vdc ───► getAEBuckConverter  ──► diae, ia, vae, Pae ───┤──► getDCBus ──► dVdc

%                                                               │

% ipe, Vdc ───► getPEMFCBoostConverter ► dipe, ip, vpe, Ppe ──┘

================================================================================

FILE: getDEMPC_AES.m

================================================================================

% =========================================================================

% getDEMPC_AES.m

% Local Distributed Economic MPC for the Alkaline Electrolyzer Subsystem

% Implements Equations 38-40 from Zhu et al., Renewable Energy 222 (2024)

%

% Inputs:

%   xa        : AES local states [iae; Vdc]               (2x1)

%   d_prev    : Previous duty cycle                       [0-1]

%   Ppv_comm  : PV power from previous timestep           [W]  (communication)

%   Ppe_comm  : PEMFC power from previous timestep        [W]  (communication)

%   is_comm   : PV bus current from previous timestep     [A]  (communication)

%   ip_comm   : PEMFC bus current from previous timestep  [A]  (communication)

%   Pload     : Load power demand                         [W]

%   Vdc_ref   : DC bus voltage reference this timestep    [V]

%

% Outputs:

%   d_ae_opt  : Optimal duty cycle                        [0-1]

%   Pae_comm  : AE power for communication to others      [W]

%   ia_comm   : AE bus input current for communication    [A]

%   NH2_comm  : H2 production rate for monitoring         [mol/s]

% =========================================================================

function [d_ae_opt, Pae_comm, ia_comm, NH2_comm] = getDEMPC_AES(xa, d_prev, ...

Ppv_comm, Ppe_comm, is_comm, ip_comm, Pload, Vdc_ref)

% ---- DEMPC parameters ----

Ts      = 40e-6;

Np      = 2;

% Weights normalized to match CMPC stability

wa1     = 400;      % Weight: power balance (surplus tracking)

wa2     = 20000;    % Weight: Vdc regulation

% ---- Generate Dynamic Adaptive Grid ----

d_ae_grid = max(0.05, min(0.65, d_prev + linspace(-0.045, 0.045, 7)'));

n_dae    = length(d_ae_grid);

n_seq    = n_dae^Np;             % 7^2 = 49 combinations

seq_table = zeros(n_seq, Np);

for j = 1:n_seq

idx1 = mod(j-1, n_dae) + 1;

idx2 = floor((j-1) / n_dae) + 1;

seq_table(j,:) = [d_ae_grid(idx1), d_ae_grid(idx2)];

end

costs   = Inf(n_seq, 1);

% ---- Evaluate cost for each candidate sequence ----

for j = 1:n_seq

xa_pred     = xa;

total_cost  = 0;

for l = 1:Np

d_ae_l   = seq_table(j, l);

% --- One-step forward prediction (local AES model) ---

xa_pred = aes_predict(xa_pred, d_ae_l, is_comm, ip_comm, Pload, Ts);

% --- AE power and H2 rate at predicted iae ---

iae_l           = max(xa_pred(1), 1e-6);

[~, Pae_l, NH2_l, ~] = getAE(iae_l);

% --- Cost terms (Eq. 38) ---

% Droop target for Surplus Power

K_vdc    = 400; % [W/V] Proportional gain

Ptarget  = max(Ppv_comm + Ppe_comm - Pload + K_vdc * (xa(2) - Vdc_ref), 0);

la1 = ((Pae_l - Ptarget) / max(Ptarget, 1))^2;

la2 = ((xa_pred(2) - Vdc_ref) / Vdc_ref)^2;

total_cost  = total_cost + wa1*la1 + wa2*la2;

end

costs(j) = total_cost;

end

% ---- Select optimal sequence ----

[~, j_opt]  = min(costs);

d_ae_opt    = seq_table(j_opt, 1);

% ---- Compute communication variables from current measured state ----

iae_now             = max(xa(1), 1e-6);

[~, Pae_comm, NH2_comm, ~] = getAE(iae_now);

ia_comm             = d_ae_opt * max(xa(1), 0);

end

% =========================================================================

% Local prediction model for AES — Forward Euler  (Eq. 13 + Eq. 10)

% xa = [iae; Vdc]

% Other subsystems' currents frozen at previous values (is_comm, ip_comm)

% =========================================================================

function xa_next = aes_predict(xa, d_ae, is_comm, ip_comm, Pload, Ts)

Lae = 1e-3;         % AE inductance   [H]

Cdc = 30e-3;        % Bus capacitance [F]

Ts_sub = Ts / 4;

xa_c   = xa;

for sub = 1:4

iae = max(xa_c(1), 0);

Vdc = max(xa_c(2), 1);

[vae, ~, ~, ~]  = getAE(max(iae, 1e-6));

il              = Pload / Vdc;

ia              = d_ae * iae;

diae    = (d_ae * Vdc - vae) / Lae;

dVdc    = (is_comm + ip_comm - ia - il) / Cdc;

xa_c        = xa_c + Ts_sub * [diae; dVdc];

xa_c(1)     = max(xa_c(1), 0);

xa_c(2)     = max(xa_c(2), 1);

end

xa_next = xa_c;

end

================================================================================

FILE: getDEMPC_AES_GWO.m

================================================================================

% =========================================================================

% getDEMPC_AES_GWO.m

% Local DEMPC for the AE Subsystem using Grey Wolf Optimizer.

% =========================================================================

function [d_ae_opt, Pae_comm, ia_comm, NH2_comm] = getDEMPC_AES_GWO(xa, ...

Ppv_comm, Ppe_comm, is_comm, ip_comm, Pload, Vdc_ref)

% ---- DEMPC parameters ----

Ts      = 40e-6;

Np      = 2;

wa1     = 400;

wa2     = 20000;

% ---- GWO parameters ----

maxIter     = 10;

wolfCount   = 8;

lowerBound  = 0.05;

upperBound  = 0.65;

% ---- Define Cost Function for GWO ----

costFunction = @(d_seq) calculate_cost(d_seq);

% ---- Run GWO to find optimal sequence ----

[best_d_seq, ~] = optimizeDutyCycleGWO(costFunction, Np, lowerBound, upperBound, maxIter, wolfCount);

d_ae_opt = best_d_seq(1);

% ---- Compute communication variables ----

iae_now             = max(xa(1), 1e-6);

[~, Pae_comm, NH2_comm, ~] = getAE(iae_now);

ia_comm             = d_ae_opt * max(xa(1), 0);

% =====================================================================

% NESTED COST FUNCTION

% =====================================================================

function total_cost = calculate_cost(d_ae_sequence)

xa_pred     = xa;

total_cost  = 0;

for l = 1:Np

d_ae_l   = d_ae_sequence(l);

% --- One-step forward prediction ---

xa_pred = aes_predict(xa_pred, d_ae_l, is_comm, ip_comm, Pload, Ts);

% --- AE power at predicted iae ---

iae_l           = max(xa_pred(1), 1e-6);

[~, Pae_l, ~, ~] = getAE(iae_l);

% --- Cost terms (Eq. 38) ---

K_vdc    = 400;

Ptarget  = max(Ppv_comm + Ppe_comm - Pload + K_vdc * (xa(2) - Vdc_ref), 0);

la1 = ((Pae_l - Ptarget) / max(Ptarget, 1))^2;

la2 = ((xa_pred(2) - Vdc_ref) / Vdc_ref)^2;

total_cost  = total_cost + wa1*la1 + wa2*la2;

end

end

end

% =========================================================================

% Local prediction model for AES — Forward Euler  (Eq. 13 + Eq. 10)

% xa = [iae; Vdc]

% Other subsystems' currents frozen at previous values (is_comm, ip_comm)

% =========================================================================

function xa_next = aes_predict(xa, d_ae, is_comm, ip_comm, Pload, Ts)

Lae = 1e-3;         % AE inductance   [H]

Cdc = 30e-3;        % Bus capacitance [F]

Ts_sub = Ts / 4;

xa_c   = xa;

for sub = 1:4

iae = max(xa_c(1), 0);

Vdc = max(xa_c(2), 1);

[vae, ~, ~, ~]  = getAE(max(iae, 1e-6));

il              = Pload / Vdc;

ia              = d_ae * iae;

diae    = (d_ae * Vdc - vae) / Lae;

dVdc    = (is_comm + ip_comm - ia - il) / Cdc;

xa_c        = xa_c + Ts_sub * [diae; dVdc];

xa_c(1)     = max(xa_c(1), 0);

xa_c(2)     = max(xa_c(2), 1);

end

xa_next = xa_c;

end

================================================================================

FILE: getDEMPC_BESS.m

================================================================================

% =========================================================================

% getDEMPC_BESS.m

% Local DMPC for Battery Energy Storage System (BESS) using Grid Search

%

% Inputs:

%   xb        : State vector [ib; SOC; Vdc]

%   db_prev   : Previous duty cycle

%   Ppv_comm  : Communicated PV power

%   Ppe_comm  : Communicated PEMFC power

%   Pae_comm  : Communicated AE power

%   is_comm   : Communicated PV current

%   ip_comm   : Communicated PEMFC current

%   ia_comm   : Communicated AE current

%   Pload     : Load power demand

%   Vdc_ref   : Reference DC bus voltage

%

% Outputs:

%   db_opt    : Optimal duty cycle

%   Pbat_comm : Expected Battery power

%   ib_comm   : Expected Battery bus current

% =========================================================================

function [db_opt, Pbat_comm, ib_comm] = getDEMPC_BESS(xb, db_prev, Ppv_comm, Ppe_comm, Pae_comm, is_comm, ip_comm, ia_comm, Pload, Vdc_ref)

% Extracted states

ib  = xb(1);

SOC = xb(2);

Vdc = xb(3);

% MPC Parameters

Ts = 40e-6;

Np = 3;

db_set = linspace(0.1, 0.9, 11); % Limits duty cycle to avoid saturation

Pbat_max = 5000; % 5 kW safety limit

% Normalized Weights (Matching CMPC scale)

w_v   = 20000; % Heavily weight Vdc regulation

w_du  = 100;

w_p   = 5000;

w_lim = 50000; % High penalty for exceeding power limit

w_soc = 50000; % High penalty for violating SOC limits

J_min = inf;

db_opt = db_prev;

for i = 1:length(db_set)

db = db_set(i);

ib_pred = ib; SOC_pred = SOC; Vdc_pred = Vdc;

J_seq = 0; db_last = db_prev;

for k = 1:Np

[Vbat, dSOC, Pbat] = getBattery(ib_pred, SOC_pred);

[dib, ib_bus] = getBidirectionalConverter(ib_pred, Vbat, Vdc_pred, db);

[dVdc, ~] = getDCBus(Vdc_pred, is_comm, ip_comm, ia_comm, ib_bus, Pload);

ib_pred = ib_pred + Ts * dib;

SOC_pred = SOC_pred + Ts * dSOC;

Vdc_pred = Vdc_pred + Ts * dVdc;

Pbal = Ppv_comm + Ppe_comm + Pbat - Pae_comm - Pload;

Pbat_penalty = max(0, abs(Pbat) - Pbat_max);

SOC_penalty = 0;

if SOC_pred >= 0.90 && Pbat < 0

SOC_penalty = abs(Pbat); % Penalize charging when full

elseif SOC_pred <= 0.20 && Pbat > 0

SOC_penalty = abs(Pbat); % Penalize discharging when empty

end

ls_v   = ((Vdc_pred - Vdc_ref) / Vdc_ref)^2;

ls_du  = (db - db_last)^2;

ls_p   = (Pbal / max(Pload, 1))^2;

ls_lim = (Pbat_penalty / Pbat_max)^2;

ls_soc = (SOC_penalty / Pbat_max)^2;

J_seq = J_seq + w_v * ls_v ...

+ w_du * ls_du ...

+ w_p * ls_p ...

+ w_lim * ls_lim ...

+ w_soc * ls_soc;

db_last = db;

end

if J_seq < J_min

J_min = J_seq;

db_opt = db;

end

end

% Communication outputs

[Vbat, ~, Pbat_comm] = getBattery(ib, SOC);

[~, ib_comm] = getBidirectionalConverter(ib, Vbat, Vdc, db_opt);

end

================================================================================

FILE: getDEMPC_BESS_GWO.m

================================================================================

% =========================================================================

% getDEMPC_BESS_GWO.m

% Local DMPC for Battery Energy Storage System (BESS) using GWO

% =========================================================================

function [db_opt, Pbat_comm, ib_comm] = getDEMPC_BESS_GWO(xb, Ppv_comm, Ppe_comm, Pae_comm, is_comm, ip_comm, ia_comm, Pload, Vdc_ref)

% Extracted states

ib  = xb(1);

SOC = xb(2);

Vdc = xb(3);

% MPC Parameters

Ts = 40e-6;

Np = 3;

% Cost function handle

costFunc = @(U_seq) evaluateCostBESS(U_seq, ib, SOC, Vdc, Ppv_comm, Ppe_comm, Pae_comm, is_comm, ip_comm, ia_comm, Pload, Vdc_ref, Ts, Np);

% GWO Parameters

dim = Np;

lowerBound = 0.1;

upperBound = 0.9;

maxIter = 20;   % Increased from 10

wolfCount = 10; % Increased from 5

% Run GWO

[bestSeq, ~] = optimizeDutyCycleGWO(costFunc, dim, lowerBound, upperBound, maxIter, wolfCount);

db_opt = bestSeq(1);

% Communication outputs

[Vbat, ~, Pbat_comm] = getBattery(ib, SOC);

[~, ib_comm] = getBidirectionalConverter(ib, Vbat, Vdc, db_opt);

end

function J = evaluateCostBESS(U_seq, ib, SOC, Vdc, Ppv, Ppe, Pae, is, ip, ia, Pload, Vdc_ref, Ts, Np)

w_v   = 20000;

w_du  = 100;

w_p   = 5000;

w_lim = 50000;

w_soc = 50000;

Pbat_max = 5000; % 5 kW safety limit

J = 0;

db_last = U_seq(1);

for k = 1:Np

db = U_seq(k);

[Vbat, dSOC, Pbat] = getBattery(ib, SOC);

[dib, ib_bus] = getBidirectionalConverter(ib, Vbat, Vdc, db);

[dVdc, ~] = getDCBus(Vdc, is, ip, ia, ib_bus, Pload);

ib = ib + Ts * dib;

SOC = SOC + Ts * dSOC;

Vdc = Vdc + Ts * dVdc;

Pbal = Ppv + Ppe + Pbat - Pae - Pload;

Pbat_penalty = max(0, abs(Pbat) - Pbat_max);

SOC_penalty = 0;

if SOC >= 0.90 && Pbat < 0

SOC_penalty = abs(Pbat); % Penalize charging when full

elseif SOC <= 0.20 && Pbat > 0

SOC_penalty = abs(Pbat); % Penalize discharging when empty

end

ls_v   = ((Vdc - Vdc_ref) / Vdc_ref)^2;

ls_du  = (db - db_last)^2;

ls_p   = (Pbal / max(Pload, 1))^2;

ls_lim = (Pbat_penalty / Pbat_max)^2;

ls_soc = (SOC_penalty / Pbat_max)^2;

J = J + w_v * ls_v + w_du * ls_du + w_p * ls_p + w_lim * ls_lim + w_soc * ls_soc;

db_last = db;

end

end

================================================================================

FILE: getDEMPC_PEMFCS.m

================================================================================

% =========================================================================

% getDEMPC_PEMFCS.m

% Local Distributed Economic MPC for the PEMFC Subsystem

% Implements Equations 41-43 from Zhu et al., Renewable Energy 222 (2024)

%

% Inputs:

%   xp        : PEMFCS local states [ipe; Vdc]            (2x1)

%   d_prev    : Previous duty cycle                       [0-1]

%   Ppv_comm  : PV power from previous timestep           [W]  (communication)

%   Pae_comm  : AE power from previous timestep           [W]  (communication)

%   is_comm   : PV bus current from previous timestep     [A]  (communication)

%   ia_comm   : AE bus current from previous timestep     [A]  (communication)

%   Pload     : Load power demand                         [W]

%   Vdc_ref   : DC bus voltage reference this timestep    [V]

%

% Outputs:

%   d_pe_opt  : Optimal duty cycle                        [0-1]

%   Ppe_comm  : PEMFC power for communication to others   [W]

%   ip_comm   : PEMFC bus current for communication       [A]

% =========================================================================

function [d_pe_opt, Ppe_comm, ip_comm] = getDEMPC_PEMFCS(xp, d_prev, ...

Ppv_comm, Pae_comm, is_comm, ia_comm, Pload, Vdc_ref)

% ---- DEMPC parameters ----

Ts      = 40e-6;

Np      = 2;

% Weights normalized to match CMPC stability

wp1     = 400;      % Weight: power balance (deficit tracking)

wp2     = 20000;    % Weight: Vdc regulation

% ---- Generate Dynamic Adaptive Grid ----

d_pe_grid = max(0.05, min(0.95, d_prev + linspace(-0.045, 0.045, 7)'));

n_dpe    = length(d_pe_grid);

n_seq    = n_dpe^Np;

seq_table = zeros(n_seq, Np);

for j = 1:n_seq

idx1 = mod(j-1, n_dpe) + 1;

idx2 = floor((j-1) / n_dpe) + 1;

seq_table(j,:) = [d_pe_grid(idx1), d_pe_grid(idx2)];

end

costs   = Inf(n_seq, 1);

% ---- Evaluate cost for each candidate sequence ----

for j = 1:n_seq

xp_pred     = xp;

total_cost  = 0;

for l = 1:Np

d_pe_l   = seq_table(j, l);

% --- One-step forward prediction (local PEMFCS model) ---

xp_pred = pemfcs_predict(xp_pred, d_pe_l, is_comm, ia_comm, Pload, Ts);

% --- PEMFC power at predicted ipe ---

ipe_l           = max(xp_pred(1), 1e-6);

[~, Ppe_l, ~]   = getPEMFC(ipe_l);

% --- Cost terms (Eq. 41) ---

% Droop target for Deficit Power

K_vdc    = 400; % [W/V] Proportional gain

Ptarget  = max(Pload + Pae_comm - Ppv_comm - K_vdc * (xp(2) - Vdc_ref), 0);

lp1 = ((Ppe_l - Ptarget) / max(Ptarget, 1))^2;

lp2 = ((xp_pred(2) - Vdc_ref) / Vdc_ref)^2;

total_cost  = total_cost + wp1*lp1 + wp2*lp2;

end

costs(j) = total_cost;

end

% ---- Select optimal sequence ----

[~, j_opt]  = min(costs);

d_pe_opt    = seq_table(j_opt, 1);

% ---- Compute communication variables from current measured state ----

ipe_now         = max(xp(1), 1e-6);

[~, Ppe_comm, ~] = getPEMFC(ipe_now);

ip_comm         = (1 - d_pe_opt) * max(xp(1), 0);

end

% =========================================================================

% Local prediction model for PEMFCS — Forward Euler (4 sub-steps)

% xp = [ipe; Vdc]

% Other subsystems' currents frozen at previous values (is_comm, ia_comm)

% =========================================================================

function xp_next = pemfcs_predict(xp, d_pe, is_comm, ia_comm, Pload, Ts)

Lpe = 1e-3;         % FC inductance   [H]

Cdc = 30e-3;        % Bus capacitance [F]

Ts_sub = Ts / 4;

xp_c   = xp;

for sub = 1:4

ipe = max(xp_c(1), 0);

Vdc = max(xp_c(2), 1);

[vpe, ~, ~]     = getPEMFC(max(ipe, 1e-6));

il              = Pload / Vdc;

ip              = (1 - d_pe) * ipe;

dipe    = (vpe - (1 - d_pe) * Vdc) / Lpe;

dVdc    = (is_comm + ip - ia_comm - il) / Cdc;

xp_c        = xp_c + Ts_sub * [dipe; dVdc];

xp_c(1)     = max(xp_c(1), 0);

xp_c(2)     = max(xp_c(2), 1);

end

xp_next = xp_c;

end

================================================================================

FILE: getDEMPC_PEMFCS_GWO.m

================================================================================

% =========================================================================

% getDEMPC_PEMFCS_GWO.m

% Local DEMPC for the PEMFC Subsystem using Grey Wolf Optimizer.

% =========================================================================

function [d_pe_opt, Ppe_comm, ip_comm] = getDEMPC_PEMFCS_GWO(xp, ...

Ppv_comm, Pae_comm, is_comm, ia_comm, Pload, Vdc_ref)

% ---- DEMPC parameters ----

Ts      = 40e-6;

Np      = 2;

wp1     = 400;

wp2     = 20000;

% ---- GWO parameters ----

maxIter     = 10;

wolfCount   = 8;

lowerBound  = 0.05;

upperBound  = 0.95;

% ---- Define Cost Function for GWO ----

costFunction = @(d_seq) calculate_cost(d_seq);

% ---- Run GWO to find optimal sequence ----

[best_d_seq, ~] = optimizeDutyCycleGWO(costFunction, Np, lowerBound, upperBound, maxIter, wolfCount);

d_pe_opt = best_d_seq(1);

% ---- Compute communication variables ----

ipe_now         = max(xp(1), 1e-6);

[~, Ppe_comm, ~] = getPEMFC(ipe_now);

ip_comm         = (1 - d_pe_opt) * max(xp(1), 0);

% =====================================================================

% NESTED COST FUNCTION

% =====================================================================

function total_cost = calculate_cost(d_pe_sequence)

xp_pred     = xp;

total_cost  = 0;

for l = 1:Np

d_pe_l   = d_pe_sequence(l);

% --- One-step forward prediction ---

xp_pred = pemfcs_predict(xp_pred, d_pe_l, is_comm, ia_comm, Pload, Ts);

% --- PEMFC power at predicted ipe ---

ipe_l           = max(xp_pred(1), 1e-6);

[~, Ppe_l, ~]   = getPEMFC(ipe_l);

% --- Cost terms (Eq. 41) ---

K_vdc    = 400;

Ptarget  = max(Pload + Pae_comm - Ppv_comm - K_vdc * (xp(2) - Vdc_ref), 0);

lp1 = ((Ppe_l - Ptarget) / max(Ptarget, 1))^2;

lp2 = ((xp_pred(2) - Vdc_ref) / Vdc_ref)^2;

total_cost  = total_cost + wp1*lp1 + wp2*lp2;

end

end

end

% =========================================================================

% Local prediction model for PEMFCS — Forward Euler (4 sub-steps)

% xp = [ipe; Vdc]

% Other subsystems' currents frozen at previous values (is_comm, ia_comm)

% =========================================================================

function xp_next = pemfcs_predict(xp, d_pe, is_comm, ia_comm, Pload, Ts)

Lpe = 1e-3;         % FC inductance   [H]

Cdc = 30e-3;        % Bus capacitance [F]

Ts_sub = Ts / 4;

xp_c   = xp;

for sub = 1:4

ipe = max(xp_c(1), 0);

Vdc = max(xp_c(2), 1);

[vpe, ~, ~]     = getPEMFC(max(ipe, 1e-6));

il              = Pload / Vdc;

ip              = (1 - d_pe) * ipe;

dipe    = (vpe - (1 - d_pe) * Vdc) / Lpe;

dVdc    = (is_comm + ip - ia_comm - il) / Cdc;

xp_c        = xp_c + Ts_sub * [dipe; dVdc];

xp_c(1)     = max(xp_c(1), 0);

xp_c(2)     = max(xp_c(2), 1);

end

xp_next = xp_c;

end

================================================================================

FILE: getDEMPC_PVS.m

================================================================================

% =========================================================================

% getDEMPC_PVS.m

% Local Distributed Economic MPC for the PV Subsystem (PVS)

% Implements Equations 33-37 from Zhu et al., Renewable Energy 222 (2024)

%

% MINLP: Enumerates all 2^Np switching sequences, selects minimum cost.

%

% Inputs:

%   xs        : PVS local states [vpv; iL; Vdc]           (3x1)

%   d_prev    : Previous duty cycle                       [0-1]

%   Pmax      : PV maximum power at current conditions    [W]

%   Ppe_comm  : PEMFC power from previous timestep        [W]  (communication)

%   Pae_comm  : AE power from previous timestep           [W]  (communication)

%   ip_comm   : PEMFC bus current from previous timestep  [A]  (communication)

%   ia_comm   : AE bus current from previous timestep     [A]  (communication)

%   Pload     : Load power demand                         [W]

%   Vdc_ref   : DC bus voltage reference this timestep    [V]  (Eq. 34)

%   G         : Solar irradiance                          [W/m^2]

%   T         : PV cell temperature                       [degC]

%

% Outputs:

%   d_s_opt   : Optimal duty cycle                        [0-1]

%   Ppv_comm  : PV power for communication to others      [W]

%   is_comm   : PV bus output current for communication   [A]

% =========================================================================

function [d_s_opt, Ppv_comm, is_comm] = getDEMPC_PVS(xs, d_prev, Pmax, ...

Ppe_comm, Pae_comm, ip_comm, ia_comm, Pload, Vdc_ref, G, T)

% ---- DEMPC parameters ----

Ts      = 40e-6;                % Sampling interval           [s]

Np      = 2;                    % Prediction horizon          [-]

% Weights normalized to match CMPC stability

ws1     = 0;                    % Power balance (delegated entirely to AE/FC)

ws2     = 800;                  % Weight: MPPT

ws3     = 20000;                % Weight: Vdc regulation

ws4     = 400;                  % Weight: Inductor current tracking

% ---- PV array config ----

Ns_pv   = 5;                    % Series modules

Np_pv   = 20;                   % Parallel strings

Voc_mod = 47.6;                 % Module open-circuit voltage [V]

vpv_max = Ns_pv * Voc_mod;     % Max PV voltage constraint   [V] (Eq. 36e)

% --- Inductor current reference ---

V_sw        = linspace(0.1, Ns_pv * Voc_mod, 500);

[~, Pp_sw]  = getPVArray(V_sw, G, T, Ns_pv, Np_pv);

[~, mi]     = max(Pp_sw);

Vmpp        = V_sw(mi);

[Impp, ~]   = getPVArray(Vmpp, G, T, Ns_pv, Np_pv);

iL_ref      = Impp;

% ---- Generate Dynamic Adaptive Grid ----

d_s_grid = max(0.05, min(0.65, d_prev + linspace(-0.045, 0.045, 7)'));

n_ds     = length(d_s_grid);

n_seq    = n_ds^Np;             % 7^2 = 49 combinations

seq_table = zeros(n_seq, Np);

for j = 1:n_seq

idx1 = mod(j-1, n_ds) + 1;

idx2 = floor((j-1) / n_ds) + 1;

seq_table(j,:) = [d_s_grid(idx1), d_s_grid(idx2)];

end

costs   = Inf(n_seq, 1);

% ---- Evaluate cost for each candidate sequence ----

for j = 1:n_seq

xs_pred     = xs;

total_cost  = 0;

feasible    = true;

for l = 1:Np

d_s_l   = seq_table(j, l);

% --- One-step forward prediction (local PVS model) ---

xs_pred = pvs_predict(xs_pred, d_s_l, ip_comm, ia_comm, ...

Pload, G, T, Ts, Ns_pv, Np_pv);

% --- Constraint check: 0 <= vpv <= vpv_max  (Gamma_cs, Eq. 37) ---

if xs_pred(1) < 0 || xs_pred(1) > vpv_max

feasible = false;

break;

end

% --- PV power at predicted state ---

vpv_l       = max(xs_pred(1), 0);

[ipv_l, ~]  = getPVArray(vpv_l, G, T, Ns_pv, Np_pv);

Ppv_l       = vpv_l * ipv_l;

% --- Cost terms (Eq. 33) ---

% Normalized identically to CMPC

ls1 = 0; % PV does not explicitly load-follow

ls2 = ((Ppv_l - Pmax) / max(Pmax, 1))^2;

ls3 = ((xs_pred(3) - Vdc_ref) / Vdc_ref)^2;

ls4 = ((xs_pred(2) - iL_ref) / max(iL_ref, 1))^2;

total_cost  = total_cost + ws1*ls1 + ws2*ls2 + ws3*ls3 + ws4*ls4;

end

if feasible

costs(j) = total_cost;

end

end

% ---- Select optimal sequence (Algorithm 1, Step 3) ----

[~, j_opt]  = min(costs);

d_s_opt     = seq_table(j_opt, 1);

% ---- Compute communication variables from current measured state ----

vpv_now         = max(xs(1), 0);

[ipv_now, ~]    = getPVArray(vpv_now, G, T, Ns_pv, Np_pv);

Ppv_comm        = vpv_now * ipv_now;

is_comm         = (1 - d_s_opt) * max(xs(2), 0);

end

% =========================================================================

% Local prediction model for PVS — Forward Euler (4 sub-steps)

% xs = [vpv; iL; Vdc]

% Other subsystems' currents frozen at previous values (ip_comm, ia_comm)

% =========================================================================

function xs_next = pvs_predict(xs, d_s, ip_comm, ia_comm, Pload, G, T, Ts, Ns_pv, Np_pv)

Cs  = 2000e-6;      % PV capacitance  [F]

Ls  = 1e-3;         % PV inductance   [H]

Cdc = 30e-3;        % Bus capacitance [F]

Ts_sub = Ts / 4;

xs_c   = xs;

for sub = 1:4

vpv = max(xs_c(1), 0);

iL  = max(xs_c(2), 0);

Vdc = max(xs_c(3), 1);

[ipv, ~]    = getPVArray(vpv, G, T, Ns_pv, Np_pv);

il          = Pload / Vdc;

is          = (1 - d_s) * iL;

dvpv    = (ipv - iL) / Cs;

diL     = (vpv - (1 - d_s) * Vdc) / Ls;

dVdc    = (is + ip_comm - ia_comm - il) / Cdc;

xs_c        = xs_c + Ts_sub * [dvpv; diL; dVdc];

xs_c(1)     = max(xs_c(1), 0);

xs_c(2)     = max(xs_c(2), 0);

xs_c(3)     = max(xs_c(3), 1);

end

xs_next = xs_c;

end

================================================================================

FILE: getDEMPC_PVS_GWO.m

================================================================================

% =========================================================================

% getDEMPC_PVS_GWO.m

% Local DEMPC for the PV Subsystem using Grey Wolf Optimizer.

%

% Replaces the grid search from the original getDEMPC_PVS.m with a

% metaheuristic optimization to find the optimal duty cycle sequence.

% =========================================================================

function [d_s_opt, Ppv_comm, is_comm] = getDEMPC_PVS_GWO(xs, Pmax, ...

Ppe_comm, Pae_comm, ip_comm, ia_comm, Pload, Vdc_ref, G, T)

% ---- DEMPC parameters ----

Ts      = 40e-6;

Np      = 2;

ws1     = 0;

ws2     = 800;

ws3     = 20000;

ws4     = 400;

% ---- PV array config ----

Ns_pv   = 5;

Np_pv   = 20;

Voc_mod = 47.6;

vpv_max = Ns_pv * Voc_mod;

% --- Inductor current reference ---

V_sw        = linspace(0.1, Ns_pv * Voc_mod, 500);

[~, Pp_sw]  = getPVArray(V_sw, G, T, Ns_pv, Np_pv);

[~, mi]     = max(Pp_sw);

Vmpp        = V_sw(mi);

[Impp, ~]   = getPVArray(Vmpp, G, T, Ns_pv, Np_pv);

iL_ref      = Impp;

% ---- GWO parameters ----

maxIter     = 10; % Keep low for speed

wolfCount   = 8;  % Keep low for speed

lowerBound  = 0.05;

upperBound  = 0.65;

% ---- Define Cost Function for GWO ----

costFunction = @(d_seq) calculate_cost(d_seq);

% ---- Run GWO to find optimal sequence ----

[best_d_seq, ~] = optimizeDutyCycleGWO(costFunction, Np, lowerBound, upperBound, maxIter, wolfCount);

d_s_opt = best_d_seq(1);

% ---- Compute communication variables ----

vpv_now         = max(xs(1), 0);

[ipv_now, ~]    = getPVArray(vpv_now, G, T, Ns_pv, Np_pv);

Ppv_comm        = vpv_now * ipv_now;

is_comm         = (1 - d_s_opt) * max(xs(2), 0);

% =====================================================================

% NESTED COST FUNCTION

% =====================================================================

function total_cost = calculate_cost(d_s_sequence)

xs_pred     = xs;

total_cost  = 0;

for l = 1:Np

d_s_l   = d_s_sequence(l);

% --- One-step forward prediction (local PVS model) ---

xs_pred = pvs_predict(xs_pred, d_s_l, ip_comm, ia_comm, ...

Pload, G, T, Ts, Ns_pv, Np_pv);

% --- Constraint check ---

if xs_pred(1) < 0 || xs_pred(1) > vpv_max

total_cost = 1e9; % High penalty for infeasible solution

return;

end

% --- PV power at predicted state ---

vpv_l       = max(xs_pred(1), 0);

[ipv_l, ~]  = getPVArray(vpv_l, G, T, Ns_pv, Np_pv);

Ppv_l       = vpv_l * ipv_l;

% --- Cost terms (Eq. 33) ---

ls1 = 0;

ls2 = ((Ppv_l - Pmax) / max(Pmax, 1))^2;

ls3 = ((xs_pred(3) - Vdc_ref) / Vdc_ref)^2;

ls4 = ((xs_pred(2) - iL_ref) / max(iL_ref, 1))^2;

total_cost  = total_cost + ws1*ls1 + ws2*ls2 + ws3*ls3 + ws4*ls4;

end

end

end

% =========================================================================

% Local prediction model for PVS — Forward Euler (4 sub-steps)

% xs = [vpv; iL; Vdc]

% Other subsystems' currents frozen at previous values (ip_comm, ia_comm)

% =========================================================================

function xs_next = pvs_predict(xs, d_s, ip_comm, ia_comm, Pload, G, T, Ts, Ns_pv, Np_pv)

Cs  = 2000e-6;      % PV capacitance  [F]

Ls  = 1e-3;         % PV inductance   [H]

Cdc = 30e-3;        % Bus capacitance [F]

Ts_sub = Ts / 4;

xs_c   = xs;

for sub = 1:4

vpv = max(xs_c(1), 0);

iL  = max(xs_c(2), 0);

Vdc = max(xs_c(3), 1);

[ipv, ~]    = getPVArray(vpv, G, T, Ns_pv, Np_pv);

il          = Pload / Vdc;

is          = (1 - d_s) * iL;

dvpv    = (ipv - iL) / Cs;

diL     = (vpv - (1 - d_s) * Vdc) / Ls;

dVdc    = (is + ip_comm - ia_comm - il) / Cdc;

xs_c        = xs_c + Ts_sub * [dvpv; diL; dVdc];

xs_c(1)     = max(xs_c(1), 0);

xs_c(2)     = max(xs_c(2), 0);

xs_c(3)     = max(xs_c(3), 1);

end

xs_next = xs_c;

end

================================================================================

FILE: getEMS.m

================================================================================

% =========================================================================

% getEMS.m

% Energy Management System

% Implements Equation 32 from Zhu et al., Renewable Energy 222 (2024) 119871

%

% Inputs:

%   Pmax  : PV maximum available power    [W]

%   Pload : Load power demand             [W]

%   SOC   : Battery State of Charge       [0 to 1]

%

% Outputs:

%   zeta_a : AES  operational mode  (1=ON, 0=OFF)

%   zeta_p : PEMFCS operational mode (1=ON, 0=OFF)

% =========================================================================

function [zeta_a, zeta_p] = getEMS(Pmax, Pload, SOC)

P_bat_max = 5000; % 5 kW power limit for the battery

SOC_min   = 0.20; % 20% minimum SOC

SOC_max   = 0.90; % 90% maximum SOC

P_net     = Pmax - Pload; % Available energy balance

% Default state: Both H2 systems idle

zeta_a = 0;

zeta_p = 0;

% Evaluate surplus conditions

if P_net > P_bat_max || (P_net > 0 && SOC >= SOC_max)

% AE absorbs if surplus > 5 kW OR if battery is full

zeta_a = 1;

end

% Evaluate deficit conditions

if P_net < -P_bat_max || (P_net < 0 && SOC <= SOC_min)

% FC supplies if deficit > 5 kW OR if battery is empty

zeta_p = 1;

end

end

================================================================================

FILE: getHydrogenTank.m

================================================================================

% =========================================================================

% getHydrogenTank.m

% Compressed Hydrogen Tank Model

%

% This function models a compressed hydrogen storage tank using the ideal

% gas law to determine the pressure based on the net flow of hydrogen.

%

% Inputs:

%   n_H2_prev : Previous amount of hydrogen in the tank (mol)

%   NH2       : Hydrogen production rate from Electrolyzer (mol/s)

%   qH2       : Hydrogen consumption rate by PEMFC (mol/s)

%   Ts        : Sampling time (s)

%

% Outputs:

%   P_tank    : Current tank pressure (Pa)

%   n_H2_new  : Updated amount of hydrogen in the tank (mol)

% =========================================================================

function [P_tank, n_H2_new] = getHydrogenTank(n_H2_prev, NH2, qH2, Ts)

% Tank Parameters

V_tank = 0.1;           % Volume of the tank (m^3)

T_tank = 298.15;        % Operating temperature (K) (approx 25 C)

R      = 8.31446;       % Universal gas constant (J/(mol*K))

% Mass Balance (Euler Integration)

n_H2_new = max(n_H2_prev + (NH2 - qH2) * Ts, 0);

% Pressure Calculation (Ideal Gas Law)

P_tank = (n_H2_new * R * T_tank) / V_tank;

end

================================================================================

FILE: getMicrogridModel.m

================================================================================

% =========================================================================

% getMicrogridModel.m

% Complete Nonlinear State-Space Model of PV/Hydrogen DC Microgrid

%

% Assembles all subsystem state equations into one function of the form:

%   xdot = f(x, u, w)

% for use in MPC prediction, open-loop simulation, and validation.

%

% Reference: Zhu et al., Renewable Energy 222 (2024) 119871

%            Equations 8, 10, 13, 28

%

% State vector x (7x1):

%   x(1) = vpv   : PV capacitor voltage          [V]

%   x(2) = iL    : PV boost inductor current      [A]

%   x(3) = iae   : AE buck inductor/AE current    [A]

%   x(4) = ipe   : PEMFC boost inductor/FC current [A]

%   x(5) = Vdc   : DC bus voltage                 [V]

%   x(6) = ib    : Battery inductor current       [A]

%   x(7) = SOC   : Battery State of Charge        [-]

%

% Control vector u (4x1):

%   u(1) = Ss    : PV boost switch signal         {0,1}

%   u(2) = Sae   : AE buck switch signal          {0,1}

%   u(3) = Spe   : PEMFC boost switch signal      {0,1}

%   u(4) = Sb    : Battery bidirectional switch   {0,1}

%

% Disturbance vector w (3x1):

%   w(1) = G     : Solar irradiance               [W/m^2]

%   w(2) = T     : PV cell temperature            [degC]

%   w(3) = Pload : Load power demand              [W]

%

% Outputs:

%   xdot  : State derivative vector (7x1)         [mixed units/s]

%   aux   : Auxiliary signals struct for monitoring and MPC cost function

%             .Ppv   : PV array power             [W]

%             .Pae   : AE power consumed          [W]

%             .Ppe   : PEMFC power output         [W]

%             .Pbat  : Battery power              [W]

%             .NH2   : H2 production rate (AE)    [mol/s]

%             .qH2   : H2 consumption rate (FC)   [mol/s]

%             .vae   : AE stack voltage            [V]

%             .vpe   : PEMFC stack voltage         [V]

%             .Vbat  : Battery stack voltage       [V]

%             .SOC   : Battery State of Charge     [-]

%             .is    : PV converter output current [A]

%             .ia    : AE converter input current  [A]

%             .ip    : PEMFC converter output curr [A]

%             .ib_bus: Battery bus current         [A]

%             .il    : Load current                [A]

%             .Pbal  : Power balance error (gen-load) [W]

% =========================================================================

function [xdot, aux] = getMicrogridModel(x, u, w)

% =====================================================================

% UNPACK STATE VECTOR

% =====================================================================

vpv     = x(1);

iL      = x(2);

iae     = x(3);

ipe     = x(4);

Vdc     = x(5);

ib      = x(6);

SOC     = x(7);

% =====================================================================

% UNPACK CONTROL VECTOR

% =====================================================================

Ss      = u(1);

Sae     = u(2);

Spe     = u(3);

Sb      = u(4);

% =====================================================================

% UNPACK DISTURBANCE VECTOR

% =====================================================================

G       = w(1);

T       = w(2);

Pload   = w(3);

% =====================================================================

% SUBSYSTEM 1: PV Boost Converter                            [Eq. 8]

% States updated: x(1)=vpv, x(2)=iL

% Also produces: is (current injected to DC bus), Ppv

% =====================================================================

[dvpv, diL, is, Ppv] = getPVBoostConverter(vpv, iL, Vdc, Ss, G, T);

% =====================================================================

% SUBSYSTEM 2: AE Buck Converter                             [Eq. 13]

% State updated: x(3)=iae

% Also produces: ia (current drawn from DC bus), vae, Pae, NH2

% =====================================================================

[diae, ia, vae, Pae, NH2] = getAEBuckConverter(iae, Vdc, Sae);

% Virtual contactor: if EMS disabled AE (Sae == 0), completely isolate it

if Sae == 0

diae = 0; ia = 0; Pae = 0; NH2 = 0;

end

% =====================================================================

% SUBSYSTEM 3: PEMFC Boost Converter                         [Eq. 28]

% State updated: x(4)=ipe

% Also produces: ip (current injected to DC bus), vpe, Ppe, qH2

% =====================================================================

[dipe, ip, vpe, Ppe, qH2] = getPEMFCBoostConverter(ipe, Vdc, Spe);

% Virtual contactor: if EMS disabled FC (Spe == 0), completely isolate it

if Spe == 0

dipe = 0; ip = 0; Ppe = 0; qH2 = 0;

end

% =====================================================================

% SUBSYSTEM 4: Battery & Bidirectional Converter

% States updated: x(6)=ib, x(7)=SOC

% Also produces: ib_bus, Pbat

% =====================================================================

[Vbat, dSOC, Pbat] = getBattery(ib, SOC);

[dib, ib_bus]      = getBidirectionalConverter(ib, Vbat, Vdc, Sb);

% =====================================================================

% SUBSYSTEM 5: DC Bus                                        [Eq. 10]

% State updated: x(5)=Vdc

% Receives is, ip, ib_bus from generators/storage and ia, il as sinks

% =====================================================================

[dVdc, il] = getDCBus(Vdc, is, ip, ia, ib_bus, Pload);

% =====================================================================

% ASSEMBLE STATE DERIVATIVE VECTOR

% =====================================================================

xdot    = [dvpv;    % dx(1)/dt

diL;     % dx(2)/dt

diae;    % dx(3)/dt

dipe;    % dx(4)/dt

dVdc;    % dx(5)/dt

dib;     % dx(6)/dt

dSOC];   % dx(7)/dt

% =====================================================================

% AUXILIARY OUTPUTS  (for MPC cost function and monitoring)

% =====================================================================

aux.Ppv     = Ppv;

aux.Pae     = Pae;

aux.Ppe     = Ppe;

aux.Pbat    = Pbat;

aux.NH2     = NH2;

aux.qH2     = qH2;

aux.vae     = vae;

aux.vpe     = vpe;

aux.Vbat    = Vbat;

aux.SOC     = SOC;

aux.is      = is;

aux.ia      = ia;

aux.ip      = ip;

aux.ib_bus  = ib_bus;

aux.il      = il;

aux.Pbal    = (Ppv + Ppe + Pbat) - (Pae + Pload);  % Power balance error [W]

end

================================================================================

FILE: getPEMFC.m

================================================================================

% =========================================================================

% getPEMFC.m

% PEMFC Stack Model

% Implements Equations 20-27 from:

% "PV/Hydrogen DC microgrid control using distributed economic MPC"

% Renewable Energy 222 (2024) 119871

%

% Inputs:

%   ipe   : PEMFC stack current (A) - scalar or vector

%

% Outputs:

%   vpe   : PEMFC stack output voltage (V)

%   Ppe   : PEMFC stack output power (W)

%   qH2   : Hydrogen consumption rate (mol/s)

% =========================================================================

function [vpe, Ppe, qH2] = getPEMFC(ipe)

% =====================================================================

% PEMFC STACK PARAMETERS  (Table 4 of reference paper)

% =====================================================================

Npe     = 300;              % Number of fuel cells in series          [-]

Tpe     = 323.15;           % Fuel cell operating temperature         [K]

PH2     = 3;                % Partial pressure of hydrogen            [atm]

PO2     = 3;                % Partial pressure of oxygen              [atm]

Ape_cm2 = 50;               % Membrane activation area                [cm^2]

Ape_m2  = Ape_cm2 * 1e-4;  % Membrane activation area                [m^2]

lambda  = 20;               % Water content of membrane               [-]

l_mem   = 5.1e-5;           % Membrane thickness                      [m]

Rc      = 3e-4;             % Contact resistance                      [Ohm]

Jmax    = 1.5;              % Maximum current density                 [A/cm^2]

% Physical constant

F       = 96485.3;          % Faraday constant                        [C/mol]

% =====================================================================

% STEP 1: Oxygen concentration at cathode-catalyst interface  [Eq. 25]

% CO2 = PO2 / (5.08e6 * exp(-498 / Tpe))

% Units: PO2 in atm, result in mol/cm^3

% =====================================================================

CO2 = PO2 / (5.08e6 * exp(-498 / Tpe));

% =====================================================================

% STEP 2: Nernst thermodynamic EMF per cell                   [Eq. 21]

% Enernst = 1.229 - 8.45e-4*(Tpe-298.15)

%         + 4.3085e-5 * Tpe * (ln(PH2) + 0.5*ln(PO2))

% =====================================================================

Enernst = 1.229 ...

- 8.45e-4  * (Tpe - 298.15) ...

+ 4.3085e-5 * Tpe * (log(PH2) + 0.5*log(PO2));

% =====================================================================

% STEP 3: Activation polarization voltage per cell             [Eq. 22]

% vact = -0.9514 + 3.12e-3*Tpe + 7.4e-5*Tpe*ln(CO2)

%        - 1.87e-4*Tpe*ln(ipe)

% Note: ipe must be > 0 for log to be valid

% =====================================================================

ipe_safe = max(ipe, 1e-6);   % Guard against log(0)

vact = -0.9514 ...

+ 3.12e-3  * Tpe ...

+ 7.4e-5   * Tpe * log(CO2) ...

- 1.87e-4  * Tpe * log(ipe_safe);

% =====================================================================

% STEP 4: Membrane resistivity (Nafion)                        [Eq. 26]

% Current density J = ipe / Ape   [A/cm^2] (Ape in cm^2)

%

% rm = 181.6 * [1 + 0.03*J + 0.062*(Tpe/303)^2 * J^2.5]

%            / [(lambda - 0.634 - 3*J) * exp(4.18*(Tpe-303)/Tpe)]

%

% Units of rm: Ohm*m  (resistivity)

% =====================================================================

J = ipe_safe / Ape_cm2;    % Current density [A/cm^2]

rm_num = 181.6 .* (1 + 0.03.*J + 0.062.*(Tpe/303).^2 .* J.^2.5);

rm_den = (lambda - 0.634 - 3.*J) .* exp(4.18*(Tpe - 303)/Tpe);

% Guard against denominator going to zero or negative

rm_den  = max(rm_den, 1e-10);

rm      = rm_num ./ rm_den;

% =====================================================================

% STEP 5: Ohmic polarization voltage per cell                  [Eq. 23]

% vohm = -ipe * (rm*l / Ape + Rc)

% rm*l/Ape: Ape in m^2, l in m, rm in Ohm*m  =>  result in Ohm

% =====================================================================

l_cm = l_mem * 100;

vohm = -ipe_safe .* (rm .* l_cm ./ Ape_cm2 + Rc);

% =====================================================================

% STEP 6: Concentration polarization voltage per cell          [Eq. 24]

% vcon = 0.016 * ln(1 - ipe / (Ape*Jmax))

% Ape in cm^2, Jmax in A/cm^2 => Ape*Jmax in A (max current limit)

% =====================================================================

I_max   = Ape_cm2 * Jmax;                      % Max stack current [A]

ratio   = ipe_safe ./ I_max;

ratio   = min(ratio, 1 - 1e-6);                % Keep argument of log < 0

vcon    = 0.016 .* log(1 - ratio);

% =====================================================================

% STEP 7: Stack output voltage                                 [Eq. 20]

% vpe = Npe * (Enernst + vact + vohm + vcon)

% Note: vact, vohm, vcon are all negative (losses)

% =====================================================================

vpe = Npe .* (Enernst + vact + vohm + vcon);

vpe = max(vpe, 0);          % Physical lower bound

% =====================================================================

% STEP 8: Stack power output                                   [Eq. 27]

% Ppe = vpe * ipe

% =====================================================================

Ppe = vpe .* ipe;

% =====================================================================

% STEP 9: Hydrogen consumption rate  (Faraday's law)

% qH2 = Npe * ipe / (2*F)    [mol/s]

% =====================================================================

qH2 = Npe .* ipe_safe ./ (2 * F);

end

================================================================================

FILE: getPEMFCBoostConverter.m

================================================================================

% =========================================================================

% getPEMFCBoostConverter.m

% PEMFC Unidirectional Boost Converter State Equations

% Implements Equation 28 from Zhu et al., Renewable Energy 222 (2024) 119871

%

% Inputs:

%   ipe   : PEMFC inductor/FC current         (A)   [state x4]

%   Vdc   : DC bus voltage                    (V)   [state x5]

%   Spe   : Switch signal                     {0,1} [control input]

%

% Outputs:

%   dipe  : d(ipe)/dt                         (A/s)

%   ip    : Output current injected to DC bus (A)

%   vpe   : PEMFC stack voltage               (V)

%   Ppe   : PEMFC output power                (W)

%   qH2   : Hydrogen consumption rate         (mol/s)

% =========================================================================

function [dipe, ip, vpe, Ppe, qH2] = getPEMFCBoostConverter(ipe, Vdc, Spe)

% --- Converter parameter (Table 4, Zhu et al.) ---

Lpe     = 01e-3;     % Boost converter inductance  [H]

% --- PEMFC stack voltage: nonlinear coupling (Eq. 20-26) ---

% Clamp current positive; getPEMFC handles its own internal guard

ipe_safe        = max(ipe, 0);

[vpe, Ppe, qH2] = getPEMFC(ipe_safe);

% --- State equation (Eq. 28) ---

% Lpe * dipe/dt = vpe(ipe) - (1 - Spe) * Vdc

dipe    = (vpe - (1 - Spe) * Vdc) / Lpe;

% --- Output current injected to DC bus ---

% ip = (1 - Spe) * ipe

ip      = (1 - Spe) * ipe_safe;

end

================================================================================

FILE: getPVArray.m

================================================================================

% =========================================================================

% pv_array_model.m

% PV Array Model - Single Diode with Rs and Rsh

% Implements Equations 2.2 - 2.8 from reference paper

%

% Inputs:

%   Vpv  : PV array terminal voltage (V) - scalar or vector

%   G    : Solar irradiance (W/m^2)

%   T    : Cell temperature (degrees C)

%   Ns   : Number of PV modules in series per string

%   Nsh  : Number of parallel strings (Nsh = Np in some notations)

%

% Outputs:

%   Ipv  : PV array output current (A)

%   Ppv  : PV array output power (W)

% =========================================================================

function [Ipv, Ppv] = getPVArray(Vpv, G, T, Ns, Nsh)

% =====================================================================

% MODULE PARAMETERS (update from datasheet if different)

% These correspond to STC: G = 1000 W/m^2, Tn = 25 C

% =====================================================================

Vocn    = 47.6;      % Module open-circuit voltage at STC (V)

Iscn    = 9.99;      % Module short-circuit current at STC (A)

Ncell   = 72;        % Number of cells per module

% Series resistance and shunt resistance (module level)

% These should ideally come from the datasheet or curve fitting.

% Typical estimates used here - update if datasheet values are available.

Rs      = 0.20;      % Series resistance per module (Ohm)   [Eq. 2.2]

Rsh     = 250.0;     % Shunt resistance per module (Ohm)    [Eq. 2.2]

% Diode and material parameters

a       = 1.30;      % Diode ideality factor (dimensionless) [Eq. 2.2]

ki      = 0.005;     % Temperature coefficient of Isc (A/C)  [Eq. 2.3]

% Typical: ~0.065% of Isc per degree C

% Update from datasheet (usually listed as mA/C or %/C)

Egap    = 1.12;      % Band gap energy of silicon (eV)        [Eq. 2.4]

% Physical constants

q       = 1.602e-19; % Electron charge (C)

k       = 1.38e-23;  % Boltzmann constant (J/K)

% Nominal temperature

Tn      = 25;        % Nominal temperature STC (degrees C)

Tn_K    = Tn + 273.15; % Nominal temperature in Kelvin

% =====================================================================

% ARRAY-LEVEL EQUIVALENT RESISTANCES         [Eq. 2.8]

% Rseq  = (Ns / Nsh) * Rs

% Rsheq = (Ns / Nsh) * Rsh

% =====================================================================

Rseq    = (Ns / Nsh) * Rs;

Rsheq   = (Ns / Nsh) * Rsh;

% =====================================================================

% TEMPERATURE CONVERSION

% =====================================================================

T_K     = T + 273.15; % Operating temperature in Kelvin

% =====================================================================

% THERMAL VOLTAGE (per cell)                 [Eq. 2.6]

% Vt = k*T/q  (single cell thermal voltage at operating temperature)

% Nominal thermal voltage at STC

% =====================================================================

Vt      = k * T_K / q;       % Operating thermal voltage (V/cell)

Vt_n    = k * Tn_K / q;      % Nominal thermal voltage at STC (V/cell)

% =====================================================================

% PHOTOCURRENT (module level)                [Eq. 2.3]

% Iph = (G/1000) * (Isc + ki * (T - Tn))

% =====================================================================

Iph     = (G / 1000) * (Iscn + ki * (T - Tn));

% =====================================================================

% NOMINAL SATURATION CURRENT at STC          [Eq. 2.5]

% Ion = Isc / (exp(Vocn / (a * Ncell * Vt_n)) - 1)

% =====================================================================

Ion     = Iscn / (exp(Vocn / (a * Ncell * Vt_n)) - 1);

% =====================================================================

% SATURATION CURRENT at operating temperature [Eq. 2.4]

% Io = Ion * (T/Tn)^3 * exp((q*Egap)/(k*a) * (1/Tn - 1/T))

% =====================================================================

Io      = Ion * (T_K / Tn_K)^3 * ...

exp( (q * Egap) / (k * a) * (1/Tn_K - 1/T_K) );

% =====================================================================

% SOLVE IMPLICIT ARRAY EQUATION via Newton-Raphson [Eq. 2.7]

%

% F(Ipv) = Ipv

%          - Nsh*Iph

%          + Nsh*Io * (exp((Vpv + Ipv*Rseq) / (a*Ns*Ncell*Vt)) - 1)

%          + (Vpv + Ipv*Rseq) / Rsheq

%        = 0

%

% dF/dIpv = 1

%           + Nsh*Io * Rseq/(a*Ns*Ncell*Vt)

%             * exp((Vpv + Ipv*Rseq) / (a*Ns*Ncell*Vt))

%           + Rseq / Rsheq

% =====================================================================

% Denominator of diode exponent for the full array

Vt_array = a * Ns * Ncell * Vt;  % [V] - thermal voltage scaled for array

% Newton-Raphson settings

maxIter  = 50;

tol      = 1e-9;

% Initial guess: ideal current (no Rs, no Rsh losses)

Ipv = Nsh * Iph - Nsh * Io * (exp(Vpv / Vt_array) - 1);

Ipv = max(0, Ipv);  % Physical lower bound

for iter = 1:maxIter

exp_term = exp((Vpv + Ipv .* Rseq) / Vt_array);

% Residual F(Ipv)

F  =  Ipv ...

- Nsh .* Iph ...

+ Nsh .* Io .* (exp_term - 1) ...

+ (Vpv + Ipv .* Rseq) ./ Rsheq;

% Jacobian dF/dIpv

dF = 1 ...

+ Nsh .* Io .* (Rseq / Vt_array) .* exp_term ...

+ Rseq / Rsheq;

% Newton update

delta   = F ./ dF;

Ipv     = Ipv - delta;

% Clamp after each iteration

Ipv     = max(0, Ipv);

% Check convergence

if max(abs(delta)) < tol

break;

end

end

% =====================================================================

% POWER OUTPUT

% =====================================================================

Ppv = Vpv .* Ipv;

end

================================================================================

FILE: getPVBoostConverter.m

================================================================================

% =========================================================================

% getPVBoostConverter.m

% PV Unidirectional Boost Converter State Equations

% Implements Equation 8 from Zhu et al., Renewable Energy 222 (2024) 119871

%

% Inputs:

%   vpv   : PV capacitor voltage              (V)   [state x1]

%   iL    : Boost inductor current            (A)   [state x2]

%   Vdc   : DC bus voltage                    (V)   [state x5]

%   Ss    : Switch signal                     {0,1} [control input]

%   G     : Solar irradiance                  (W/m^2)

%   T     : Cell temperature                  (degC)

%

% Outputs:

%   dvpv  : d(vpv)/dt                         (V/s)

%   diL   : d(iL)/dt                          (A/s)

%   is    : Output current injected to DC bus (A)

%   Ppv   : PV array instantaneous power      (W)

% =========================================================================

function [dvpv, diL, is, Ppv] = getPVBoostConverter(vpv, iL, Vdc, Ss, G, T)

% --- Converter parameters (Table 4, Zhu et al.) ---

Cs  = 2000e-6;      % PV side capacitance     [F]

Ls  = 01e-3;       % Boost inductance         [H]

% --- PV array current: nonlinear coupling (Eq. 8 comment + Eq. 1) ---

% getPVArray returns [Ipv, Ppv] at the operating voltage vpv

% vpv must be clamped positive before passing to getPVArray

vpv_safe        = max(vpv, 0);

[ipv, Ppv]      = getPVArray(vpv_safe, G, T, 5, 20);

% --- State equations (Eq. 8) ---

% Cs * dvpv/dt = ipv(vpv) - iL

dvpv    = (ipv - iL) / Cs;

% Ls * diL/dt  = vpv - (1 - Ss) * Vdc

diL     = (vpv - (1 - Ss) * Vdc) / Ls;

% --- Output current injected to DC bus ---

% is = (1 - Ss) * iL

is      = (1 - Ss) * iL;

end

================================================================================

FILE: optimizeDutyCycleGWO.m

================================================================================

% =========================================================================

% optimizeDutyCycleGWO.m

% Grey Wolf Optimizer for Duty Cycle Optimization

%

% This function finds the optimal duty cycle sequence that minimizes a

% given cost function using the Grey Wolf Optimizer (GWO) metaheuristic.

%

% Inputs:

%   costFunc    : Function handle to the objective/cost function.

%                 The cost function must accept a row vector of duty cycles

%                 and return a scalar cost.

%   dim         : The dimension of the problem (number of duty cycles in a

%                 sequence, i.e., the prediction horizon Np).

%   lowerBound  : Lower bound for each duty cycle (scalar).

%   upperBound  : Upper bound for each duty cycle (scalar).

%   maxIter     : Maximum number of iterations for the optimizer.

%   wolfCount   : Number of search agents (wolves).

%

% Outputs:

%   bestSequence : The optimal sequence of duty cycles found (1 x dim).

%   bestCost     : The minimum cost value corresponding to bestSequence.

% =========================================================================

function [bestSequence, bestCost] = optimizeDutyCycleGWO(costFunc, dim, lowerBound, upperBound, maxIter, wolfCount)

% Initialize the positions of the alpha, beta, and delta wolves

Alpha_pos   = zeros(1, dim);

Alpha_score = inf;

Beta_pos    = zeros(1, dim);

Beta_score  = inf;

Delta_pos   = zeros(1, dim);

Delta_score = inf;

% Initialize the wolf pack positions randomly

Positions = lowerBound + (upperBound - lowerBound) * rand(wolfCount, dim);

% Main loop

for iter = 1:maxIter

for i = 1:wolfCount

% Return wolves that go outside the search space

Positions(i,:) = max(Positions(i,:), lowerBound);

Positions(i,:) = min(Positions(i,:), upperBound);

% Calculate objective function for each wolf

fitness = costFunc(Positions(i,:));

% Update Alpha, Beta, and Delta

if fitness < Alpha_score, Alpha_score = fitness; Alpha_pos = Positions(i,:); end

if fitness > Alpha_score && fitness < Beta_score, Beta_score = fitness; Beta_pos = Positions(i,:); end

if fitness > Alpha_score && fitness > Beta_score && fitness < Delta_score, Delta_score = fitness; Delta_pos = Positions(i,:); end

end

a = 2 - iter * (2 / maxIter); % Linearly decreasing 'a' from 2 to 0

% Update the position of the wolves

for i = 1:wolfCount

r1 = rand(1, dim); r2 = rand(1, dim); A1 = 2 * a * r1 - a; C1 = 2 * r2;

D_alpha = abs(C1 .* Alpha_pos - Positions(i,:)); X1 = Alpha_pos - A1 .* D_alpha;

r1 = rand(1, dim); r2 = rand(1, dim); A2 = 2 * a * r1 - a; C2 = 2 * r2;

D_beta = abs(C2 .* Beta_pos - Positions(i,:)); X2 = Beta_pos - A2 .* D_beta;

r1 = rand(1, dim); r2 = rand(1, dim); A3 = 2 * a * r1 - a; C3 = 2 * r2;

D_delta = abs(C3 .* Delta_pos - Positions(i,:)); X3 = Delta_pos - A3 .* D_delta;

Positions(i,:) = (X1 + X2 + X3) / 3;

end

end

bestSequence = Alpha_pos;

bestCost = Alpha_score;

end

================================================================================

FILE: runDEMPC.m

================================================================================

% =========================================================================

% runDEMPC.m

% Main DEMPC Simulation Loop for PV/Hydrogen DC Microgrid

%

% Coordinates three local controllers with EMS, following the DEMPC

% application procedure in Section 3.4, Zhu et al. (2024)

%

% Inputs:

%   x_init        : Initial state [vpv; iL; iae; ipe; Vdc]   (5x1)

%   t_sim         : Total simulation time                     [s]

%   G_profile     : Function handle — irradiance G(t)         [W/m^2]

%   T_profile     : Function handle — temperature T(t)        [degC]

%   Pload_profile : Function handle — load power Pload(t)     [W]

%

% Output:

%   results : Struct containing all logged signals

% =========================================================================

function results = runDEMPC(x_init, t_sim, G_profile, T_profile, Pload_profile)

% ---- Simulation parameters ----

Ts      = 40e-6;                    % Sampling interval     [s]   (Table 3)

Ns_ref  = 10;                       % Reference horizon     [-]   (Table 3)

Vdc_set = 300;                      % DC bus setpoint       [V]   (Table 4)

N_steps = round(t_sim / Ts);        % Total simulation steps

% ---- PV array config (for Pmax sweep) ----

Ns_pv   = 5;

Np_pv   = 20;

Voc_mod = 47.6;

V_sweep = linspace(0.1, Ns_pv * Voc_mod, 500);  % 500-pt MPP sweep

% =====================================================================

% INITIALIZE STATE AND COMMUNICATION VARIABLES

% =====================================================================

Vdc_ref = Vdc_set;

x       = x_init;

d_prev  = [0.35; 0.22; 0.30; 0.5]; % Adaptive grid initial guesses

% Initialize communication variables from initial conditions

G0  = G_profile(0);   T0 = T_profile(0);

vpv0 = x(1);   iL0 = x(2);   iae0 = x(3);   ipe0 = x(4);

Vdc0 = x(5);   ib0 = x(6);   SOC0 = x(7);

[ipv0, ~]           = getPVArray(max(vpv0,0), G0, T0, Ns_pv, Np_pv);

Ppv_comm            = max(vpv0, 0) * ipv0;

is_comm             = (1 - d_prev(1))  * max(iL0,  0);

[~, Pae_comm, NH2_init, ~] = getAE(max(iae0, 1e-6));

ia_comm             = d_prev(2) * max(iae0, 0);

[~, Ppe_comm, ~]    = getPEMFC(max(ipe0, 1e-6));

ip_comm             = (1 - d_prev(3)) * max(ipe0, 0);

[Vbat0, ~, Pbat_comm] = getBattery(ib0, SOC0);

[~, ib_comm]          = getBidirectionalConverter(ib0, Vbat0, Vdc0, d_prev(4));

% ---- Initialize Hydrogen Tank ----

V_tank   = 0.1;                                 % Volume [m^3]

T_tank   = 298.15;                              % Temp [K]

R_gas    = 8.31446;                             % Gas constant

P_tank0  = 10 * 1e5;                            % Init pressure (10 bar)

n_H2     = P_tank0 * V_tank / (R_gas * T_tank); % Init moles

% =====================================================================

% PRE-ALLOCATE RESULTS

% =====================================================================

t_vec       = (0:N_steps-1)' * Ts;

X           = zeros(N_steps, 7);

U           = zeros(N_steps, 4);

Ppv_log     = zeros(N_steps, 1);

Pae_log     = zeros(N_steps, 1);

Ppe_log     = zeros(N_steps, 1);

Pmax_log    = zeros(N_steps, 1);

Pbal_log    = zeros(N_steps, 1);

NH2_log     = zeros(N_steps, 1);

qH2_log     = zeros(N_steps, 1);

Vdc_ref_log = zeros(N_steps, 1);

zeta_log    = zeros(N_steps, 2);    % [zeta_a, zeta_p]

Ptank_log   = zeros(N_steps, 1);

Pbat_log    = zeros(N_steps, 1);

SOC_log     = zeros(N_steps, 1);

fprintf('Running DEMPC: %d steps (Ts=%.0f µs, t_sim=%.3f s)...\n', ...

N_steps, Ts*1e6, t_sim);

report_every = max(1, round(N_steps/20));

% =====================================================================

% MAIN SIMULATION LOOP  (Section 3.4, Zhu et al.)

% =====================================================================

for k = 1:N_steps

t_now   = (k-1) * Ts;

% --- Current disturbances ---

G_now   = G_profile(t_now);

T_now   = T_profile(t_now);

Pload   = Pload_profile(t_now);

% --- Unpack current states ---

vpv = x(1);   iL  = x(2);

iae = x(3);   ipe = x(4);

Vdc = x(5);   ib  = x(6);

SOC = x(7);

% =================================================================

% STEP 1: Compute PV maximum power Pmax  (Eq. 7)

% =================================================================

[~, Pp_sw]  = getPVArray(V_sweep, G_now, T_now, Ns_pv, Np_pv);

Pmax        = max(Pp_sw);

% =================================================================

% STEP 2: EMS — determine operational modes  (Eq. 32)

% =================================================================

[zeta_a, zeta_p] = getEMS(Pmax, Pload, SOC);

% =================================================================

% STEP 3: Vdc reference — gradual approach  (Eq. 34)

% =================================================================

Vdc_ref = Vdc_ref + (1/Ns_ref) * (Vdc_set - Vdc_ref);

% Vdc = Vdc_ref + (1/Ns_ref) * (Vdc_set - Vdc);

% =================================================================

% STEP 4: Local DEMPC for PVS  (always active)

% =================================================================

xs  = [vpv; iL; Vdc];

Pload_eff = Pload - Pbat_comm; % Effective load for legacy controllers

[d_s_opt, Ppv_comm, is_comm] = getDEMPC_PVS(xs, d_prev(1), Pmax, ...

Ppe_comm, Pae_comm, ip_comm, ia_comm, Pload_eff, Vdc_ref, G_now, T_now);

% =================================================================

% STEP 5: Local DEMPC for BESS (always active)

% =================================================================

xb  = [ib; SOC; Vdc];

Pae_comm_temp = 0;

Ppe_comm_temp = 0;

[d_b_opt, Pbat_comm, ib_comm] = getDEMPC_BESS(xb, d_prev(4), ...

Ppv_comm, Ppe_comm_temp, Pae_comm_temp, is_comm, ip_comm, ia_comm, Pload, Vdc_ref);

% =================================================================

% STEP 6: Local DEMPC for AES  (only when EMS activates it)

% When off: force iae=0, reset communication to zero

% =================================================================

if zeta_a == 1

xa      = [iae; Vdc];

[d_ae_opt, Pae_comm, ia_comm, ~] = getDEMPC_AES(xa, d_prev(2), ...

Ppv_comm, Ppe_comm, is_comm, ip_comm, Pload_eff, Vdc_ref);

else

d_ae_opt = 0;

Pae_comm = 0;

ia_comm  = 0;

end

% =================================================================

% STEP 7: Local DEMPC for PEMFCS  (only when EMS activates it)

% When off: force ipe=0, reset communication to zero

% =================================================================

if zeta_p == 1

xp      = [ipe; Vdc];

[d_pe_opt, Ppe_comm, ip_comm] = getDEMPC_PEMFCS(xp, d_prev(3), ...

Ppv_comm, Pae_comm, is_comm, ia_comm, Pload_eff, Vdc_ref);

else

d_pe_opt = 0;

Ppe_comm = 0;

ip_comm  = 0;

end

% =================================================================

% STEP 8: Apply optimal control to plant  (forward Euler integration)

% =================================================================

u   = [d_s_opt; d_ae_opt; d_pe_opt; d_b_opt];

w   = [G_now; T_now; Pload];

Ts_sub  = Ts / 4;

for sub = 1:4

[xdot, aux] = getMicrogridModel(x, u, w);

x           = x + Ts_sub * xdot;

x(1)        = max(x(1), 0);

x(2)        = max(x(2), 0);

x(3)        = max(x(3), 0);

x(4)        = max(x(4), 0);

x(5)        = max(x(5), 1);

x(7)        = max(min(x(7), 1), 0); % Bound SOC between 0 and 1

end

% When subsystem is off, forcefully zero the state (contactor open)

if zeta_a == 0

x(3) = 0;

end

if zeta_p == 0

x(4) = 0;

end

% =================================================================

% STEP 9: Update previous switch signals

% =================================================================

d_prev = u;

% =================================================================

% STEP 10: Log results

% =================================================================

% Update Tank state

[P_tank, n_H2]  = getHydrogenTank(n_H2, aux.NH2, aux.qH2, Ts);

X(k,:)          = x';

U(k,:)          = u';

Ppv_log(k)      = aux.Ppv;

Pae_log(k)      = aux.Pae;

Ppe_log(k)      = aux.Ppe;

Pbal_log(k)     = aux.Pbal;

NH2_log(k)      = aux.NH2;

qH2_log(k)      = aux.qH2;

Pmax_log(k)     = Pmax;

Vdc_ref_log(k)  = Vdc_ref;

zeta_log(k,:)   = [zeta_a, zeta_p];

Ptank_log(k)    = P_tank;

Pbat_log(k)     = aux.Pbat;

SOC_log(k)      = x(7);

% Progress report

if mod(k, report_every) == 0

Perror = aux.Ppv + aux.Ppe + aux.Pbat - Pload - aux.Pae;

fprintf('  %3d%% | t=%.4fs | Vdc=%.1fV | Ppv=%.2fkW | Pbat=%.2fkW | Pae=%.2fkW | Pfc=%.2fkW | Perror=%.0fW | EMS=[%d,%d]\n', ...

round(100*k/N_steps), t_now, x(5), aux.Ppv/1000, aux.Pbat/1000, aux.Pae/1000, aux.Ppe/1000, Perror, zeta_a, zeta_p);

end

end

% =====================================================================

% PACKAGE RESULTS

% =====================================================================

results.t       = t_vec;

results.X       = X;           % [vpv, iL, iae, ipe, Vdc, ib, SOC]

results.U       = U;           % [Ss, Sae, Spe, Sb]

results.Ppv     = Ppv_log;

results.Pae     = Pae_log;

results.Ppe     = Ppe_log;

results.Pmax    = Pmax_log;

results.Pbal    = Pbal_log;

results.NH2     = NH2_log;

results.qH2     = qH2_log;

results.Vdc_ref = Vdc_ref_log;

results.zeta    = zeta_log;

results.Ptank   = Ptank_log;

results.Pbat    = Pbat_log;

results.SOC     = SOC_log;

results.Ts      = Ts;

results.total_H2_prod = sum(NH2_log) * Ts; % Total H2 produced [mol]

results.total_H2_cons = sum(qH2_log) * Ts; % Total H2 consumed [mol]

fprintf('Simulation complete.\n');

end

================================================================================

FILE: runDEMPC_GWO.m

================================================================================

% =========================================================================

% runDEMPC_GWO.m

% Main DEMPC Simulation Loop for PV/Hydrogen DC Microgrid

%

% Coordinates three local controllers with EMS, following the DEMPC

% application procedure in Section 3.4, Zhu et al. (2024)

%

% Inputs:

%   x_init        : Initial state [vpv; iL; iae; ipe; Vdc]   (5x1)

%   t_sim         : Total simulation time                     [s]

%   G_profile     : Function handle — irradiance G(t)         [W/m^2]

%   T_profile     : Function handle — temperature T(t)        [degC]

%   Pload_profile : Function handle — load power Pload(t)     [W]

%

% Output:

%   results : Struct containing all logged signals

% =========================================================================

function results = runDEMPC_GWO(x_init, t_sim, G_profile, T_profile, Pload_profile)

% ---- Simulation parameters ----

Ts      = 40e-6;                    % Sampling interval     [s]   (Table 3)

Ns_ref  = 10;                       % Reference horizon     [-]   (Table 3)

Vdc_set = 300;                      % DC bus setpoint       [V]   (Table 4)

N_steps = round(t_sim / Ts);        % Total simulation steps

% ---- PV array config (for Pmax sweep) ----

Ns_pv   = 5;

Np_pv   = 20;

Voc_mod = 47.6;

V_sweep = linspace(0.1, Ns_pv * Voc_mod, 500);  % 500-pt MPP sweep

% =====================================================================

% INITIALIZE STATE AND COMMUNICATION VARIABLES

% =====================================================================

Vdc_ref = Vdc_set;

x       = x_init;

d_prev  = [0.35; 0.22; 0.30; 0.5]; % Adaptive grid initial guesses

% Initialize communication variables from initial conditions

G0  = G_profile(0);   T0 = T_profile(0);

vpv0 = x(1);   iL0 = x(2);   iae0 = x(3);   ipe0 = x(4);

Vdc0 = x(5);   ib0 = x(6);   SOC0 = x(7);

[ipv0, ~]           = getPVArray(max(vpv0,0), G0, T0, Ns_pv, Np_pv);

Ppv_comm            = max(vpv0, 0) * ipv0;

is_comm             = (1 - d_prev(1))  * max(iL0,  0);

[~, Pae_comm, NH2_init, ~] = getAE(max(iae0, 1e-6));

ia_comm             = d_prev(2) * max(iae0, 0);

[~, Ppe_comm, ~]    = getPEMFC(max(ipe0, 1e-6));

ip_comm             = (1 - d_prev(3)) * max(ipe0, 0);

[Vbat0, ~, Pbat_comm] = getBattery(ib0, SOC0);

[~, ib_comm]          = getBidirectionalConverter(ib0, Vbat0, Vdc0, d_prev(4));

% ---- Initialize Hydrogen Tank ----

V_tank   = 0.1;                                 % Volume [m^3]

T_tank   = 298.15;                              % Temp [K]

R_gas    = 8.31446;                             % Gas constant

P_tank0  = 10 * 1e5;                            % Init pressure (10 bar)

n_H2     = P_tank0 * V_tank / (R_gas * T_tank); % Init moles

% =====================================================================

% PRE-ALLOCATE RESULTS

% =====================================================================

t_vec       = (0:N_steps-1)' * Ts;

X           = zeros(N_steps, 7);

U           = zeros(N_steps, 4);

Ppv_log     = zeros(N_steps, 1);

Pae_log     = zeros(N_steps, 1);

Ppe_log     = zeros(N_steps, 1);

Pmax_log    = zeros(N_steps, 1);

Pbal_log    = zeros(N_steps, 1);

NH2_log     = zeros(N_steps, 1);

qH2_log     = zeros(N_steps, 1);

Vdc_ref_log = zeros(N_steps, 1);

zeta_log    = zeros(N_steps, 2);    % [zeta_a, zeta_p]

Ptank_log   = zeros(N_steps, 1);

Pbat_log    = zeros(N_steps, 1);

SOC_log     = zeros(N_steps, 1);

fprintf('Running GWO-DEMPC: %d steps (Ts=%.0f µs, t_sim=%.3f s)...\n', ...

N_steps, Ts*1e6, t_sim);

report_every = max(1, round(N_steps/20));

% =====================================================================

% MAIN SIMULATION LOOP  (Section 3.4, Zhu et al.)

% =====================================================================

for k = 1:N_steps

t_now   = (k-1) * Ts;

% --- Current disturbances ---

G_now   = G_profile(t_now);

T_now   = T_profile(t_now);

Pload   = Pload_profile(t_now);

% --- Unpack current states ---

vpv = x(1);   iL  = x(2);

iae = x(3);   ipe = x(4);

Vdc = x(5);   ib  = x(6);

SOC = x(7);

% =================================================================

% STEP 1: Compute PV maximum power Pmax  (Eq. 7)

% =================================================================

[~, Pp_sw]  = getPVArray(V_sweep, G_now, T_now, Ns_pv, Np_pv);

Pmax        = max(Pp_sw);

% =================================================================

% STEP 2: EMS — determine operational modes  (Eq. 32)

% =================================================================

[zeta_a, zeta_p] = getEMS(Pmax, Pload, SOC);

% =================================================================

% STEP 3: Vdc reference — gradual approach  (Eq. 34)

% =================================================================

Vdc_ref = Vdc_ref + (1/Ns_ref) * (Vdc_set - Vdc_ref);

% Vdc = Vdc_ref + (1/Ns_ref) * (Vdc_set - Vdc);

% =================================================================

% STEP 4: Local DEMPC for PVS  (always active)

% =================================================================

xs  = [vpv; iL; Vdc];

Pload_eff = Pload - Pbat_comm; % Effective load for legacy controllers

[d_s_opt, Ppv_comm, is_comm] = getDEMPC_PVS_GWO(xs, Pmax, ...

Ppe_comm, Pae_comm, ip_comm, ia_comm, Pload_eff, Vdc_ref, G_now, T_now);

% =================================================================

% STEP 5: Local DEMPC for BESS (always active)

% =================================================================

xb  = [ib; SOC; Vdc];

Pae_comm_temp = 0;

Ppe_comm_temp = 0;

[d_b_opt, Pbat_comm, ib_comm] = getDEMPC_BESS_GWO(xb, ...

Ppv_comm, Ppe_comm_temp, Pae_comm_temp, is_comm, ip_comm, ia_comm, Pload, Vdc_ref);

% =================================================================

% STEP 6: Local DEMPC for AES  (only when EMS activates it)

% When off: force iae=0, reset communication to zero

% =================================================================

if zeta_a == 1

xa      = [iae; Vdc];

[d_ae_opt, Pae_comm, ia_comm, ~] = getDEMPC_AES_GWO(xa, ...

Ppv_comm, Ppe_comm, is_comm, ip_comm, Pload_eff, Vdc_ref);

else

d_ae_opt = 0;

Pae_comm = 0;

ia_comm  = 0;

end

% =================================================================

% STEP 7: Local DEMPC for PEMFCS  (only when EMS activates it)

% When off: force ipe=0, reset communication to zero

% =================================================================

if zeta_p == 1

xp      = [ipe; Vdc];

[d_pe_opt, Ppe_comm, ip_comm] = getDEMPC_PEMFCS_GWO(xp, ...

Ppv_comm, Pae_comm, is_comm, ia_comm, Pload_eff, Vdc_ref);

else

d_pe_opt = 0;

Ppe_comm = 0;

ip_comm  = 0;

end

% =================================================================

% STEP 8: Apply optimal control to plant  (forward Euler integration)

% =================================================================

u   = [d_s_opt; d_ae_opt; d_pe_opt; d_b_opt];

w   = [G_now; T_now; Pload];

Ts_sub  = Ts / 4;

for sub = 1:4

[xdot, aux] = getMicrogridModel(x, u, w);

x           = x + Ts_sub * xdot;

x(1)        = max(x(1), 0);

x(2)        = max(x(2), 0);

x(3)        = max(x(3), 0);

x(4)        = max(x(4), 0);

x(5)        = max(x(5), 1);

x(7)        = max(min(x(7), 1), 0); % Bound SOC between 0 and 1

end

% When subsystem is off, forcefully zero the state (contactor open)

if zeta_a == 0

x(3) = 0;

end

if zeta_p == 0

x(4) = 0;

end

% =================================================================

% STEP 9: Update previous switch signals

% =================================================================

d_prev = u;

% =================================================================

% STEP 10: Log results

% =================================================================

% Update Tank state

[P_tank, n_H2]  = getHydrogenTank(n_H2, aux.NH2, aux.qH2, Ts);

X(k,:)          = x';

U(k,:)          = u';

Ppv_log(k)      = aux.Ppv;

Pae_log(k)      = aux.Pae;

Ppe_log(k)      = aux.Ppe;

Pbal_log(k)     = aux.Pbal;

NH2_log(k)      = aux.NH2;

qH2_log(k)      = aux.qH2;

Pmax_log(k)     = Pmax;

Vdc_ref_log(k)  = Vdc_ref;

zeta_log(k,:)   = [zeta_a, zeta_p];

Ptank_log(k)    = P_tank;

Pbat_log(k)     = aux.Pbat;

SOC_log(k)      = x(7);

% Progress report

if mod(k, report_every) == 0

Perror = aux.Ppv + aux.Ppe + aux.Pbat - Pload - aux.Pae;

fprintf('  %3d%% | t=%.4fs | Vdc=%.1fV | Ppv=%.2fkW | Pbat=%.2fkW | Pae=%.2fkW | Pfc=%.2fkW | Perror=%.0fW | EMS=[%d,%d]\n', ...

round(100*k/N_steps), t_now, x(5), aux.Ppv/1000, aux.Pbat/1000, aux.Pae/1000, aux.Ppe/1000, Perror, zeta_a, zeta_p);

end

end

% =====================================================================

% PACKAGE RESULTS

% =====================================================================

results.t       = t_vec;

results.X       = X;           % [vpv, iL, iae, ipe, Vdc, ib, SOC]

results.U       = U;           % [Ss, Sae, Spe, Sb]

results.Ppv     = Ppv_log;

results.Pae     = Pae_log;

results.Ppe     = Ppe_log;

results.Pmax    = Pmax_log;

results.Pbal    = Pbal_log;

results.NH2     = NH2_log;

results.qH2     = qH2_log;

results.Vdc_ref = Vdc_ref_log;

results.zeta    = zeta_log;

results.Ptank   = Ptank_log;

results.Pbat    = Pbat_log;

results.SOC     = SOC_log;

results.Ts      = Ts;

results.total_H2_prod = sum(NH2_log) * Ts; % Total H2 produced [mol]

results.total_H2_cons = sum(qH2_log) * Ts; % Total H2 consumed [mol]

fprintf('Simulation complete.\n');

end

================================================================================

FILE: stepMicrogridEnv.m

================================================================================

% =========================================================================

% stepMicrogridEnv.m

% Reinforcement Learning Environment - Macro Power Model

%

% Inputs:

%   state  : [SOC; Ptank (Pa)]

%   action : 1 = EMS Idle, 2 = AE ON, 3 = FC ON

%   prev_action : The action taken in the previous step

%   Ppv    : Current PV Power (W)

%   Pload  : Current Load Power (W)

%   dt     : Time step duration (seconds)

%

% Outputs:

%   next_state : [SOC_new; Ptank_new]

%   reward     : Reward signal for the RL agent

%   is_done    : Boolean flag if episode terminates (e.g. failure)

%   info       : Struct with power logging details

% =========================================================================

function [next_state, reward, is_done, info] = stepMicrogridEnv(state, action, prev_action, Ppv, Pload, dt)

% Unpack State

SOC   = state(1);

Ptank = state(2);

% System Parameters

P_bat_max = 5000;                % Battery power limit (W)

E_bat_max = 240 * 100 * 3600;    % Battery capacity (Joules) -> 240V * 100Ah

V_tank    = 0.1;                 % Tank volume (m^3)

T_tank    = 298.15;              % Tank temp (K)

R_gas     = 8.31446;             % Universal gas constant

% Calculate initial net power

P_net = Ppv - Pload;

P_ae = 0; P_fc = 0;

% Apply Action (Hydrogen Systems)

if action == 2 && P_net > 0

% AE ON: Greedily absorb surplus up to ~20kW limit

P_ae = min(P_net, 20000);

elseif action == 3 && P_net < 0

% FC ON: Greedily supply deficit up to ~15kW limit

P_fc = min(abs(P_net), 15000);

end

% Battery handles the remainder

P_rem = P_net - P_ae + P_fc;

P_bat = min(max(-P_rem, -P_bat_max), P_bat_max); % Positive = Discharging

% Calculate any dropped load or wasted power (Mismatch)

P_mismatch = abs(P_rem + P_bat);

% Update States

SOC_new = SOC - (P_bat * dt) / E_bat_max;

% Simplified Hydrogen Molar Conversion (approx from your models)

NH2 = (P_ae / 1000) * 0.0006; % ~0.6 mmol per kW

qH2 = (P_fc / 1000) * 0.0012; % ~1.2 mmol per kW

n_H2 = (Ptank * V_tank) / (R_gas * T_tank);

n_H2_new = max(n_H2 + (NH2 - qH2) * dt, 0);

Ptank_new = (n_H2_new * R_gas * T_tank) / V_tank;

next_state = [SOC_new; Ptank_new];

% =====================================================================

% REWARD FUNCTION (The AI's Objective)

% =====================================================================

reward = 0;

reward = reward - (P_mismatch / 1000) * 5; % -5 points for every kW dropped

if SOC_new < 0.20 || SOC_new > 0.90

reward = reward - 50; % Penalty for leaving safe SOC bounds

end

if action ~= prev_action

reward = reward - 10; % Penalty for switching states (Chattering)

end

is_done = (SOC_new <= 0.01 || SOC_new >= 0.99); % Fatal failure

% Logging info

info = struct('Pbat', P_bat, 'Pae', P_ae, 'Pfc', P_fc, 'Pmiss', P_mismatch);

end

================================================================================

FILE: test_comparison.m

================================================================================

%%

% =========================================================================

% test_comparison.m

%

% Compares the performance of the original DMPC (grid search) against

% the new DMPC using Grey Wolf Optimization (GWO).

%

% This script will:

% 1. Define the simulation scenario (initial state, duration, disturbances).

% 2. Run the original 'runDEMPC' simulation.

% 3. Run the new 'runDEMPC_GWO' simulation.

% 4. Plot the results of both simulations on the same axes for comparison.

% =========================================================================

%clear; clc; close all;

% ---- Simulation Scenario ----

t_sim = 0.8; % Extended to 800ms for full 4-stage profile

% ---- Disturbance Profiles (matching test_DEMPC.m) ----

% Irradiance: 1000 -> ramp to 1100 -> hold 1100 -> step to 1000 (0.2s intervals)

G_profile = @(t) 1000 .* (t < 0.2) + ...

(1000 + 100*(t-0.2)/0.2) .* (t >= 0.2 & t < 0.4) + ...

1100 .* (t >= 0.4 & t < 0.6) + ...

1000 .* (t >= 0.6);

% Temperature: fixed

T_profile = @(t) 25;

% Load: 20kW -> ramp to 40kW -> hold 40kW -> step to 50kW (0.2s intervals)

Pload_profile = @(t) 20000 .* (t < 0.2) + ...

(20000 + 20000*(t-0.2)/0.2) .* (t >= 0.2 & t < 0.4) + ...

40000 .* (t >= 0.4 & t < 0.6) + ...

50000 .* (t >= 0.6);

% ---- Initial State Calculation (matching test_DEMPC.m) ----

Ns_pv   = 5;    Np_pv = 20;

Voc_mod = 47.6;

G0      = G_profile(0);

T0      = T_profile(0);

Vdc_nom = 300;

% Find MPP at t=0 for initial vpv and iL

V_sw    = linspace(0.1, Ns_pv*Voc_mod, 500);

[~, Pp] = getPVArray(V_sw, G0, T0, Ns_pv, Np_pv);

[~, mi] = max(Pp);

vpv0    = V_sw(mi);

[Impp0, Ppv0] = getPVArray(vpv0, G0, T0, Ns_pv, Np_pv);

% Compute initial AE current for surplus power

P_surplus_0 = max(Ppv0 - Pload_profile(0), 0);

iae_sw      = linspace(0, 500, 5000);

[vae_sw, ~, ~, ~] = getAE(max(iae_sw,1e-6));

[~, idx_ae] = min(abs(vae_sw .* iae_sw - P_surplus_0));

iae_init    = iae_sw(idx_ae);

ib_init  = 0;

SOC_init = 0.8; % 80% Initial SOC

x_init   = [vpv0; Impp0; iae_init; 0; Vdc_nom; ib_init; SOC_init];

% =========================================================================

% RUN SIMULATIONS

% =========================================================================

% Run the original DMPC with grid search

fprintf('--- Starting Original DMPC Simulation ---\n');

results_dempc = runDEMPC(x_init, t_sim, G_profile, T_profile, Pload_profile);

fprintf('--- Original DMPC Simulation Complete ---\n\n');

% Run the new DMPC with Grey Wolf Optimizer

fprintf('--- Starting GWO-DMPC Simulation ---\n');

results_gwo = runDEMPC_GWO(x_init, t_sim, G_profile, T_profile, Pload_profile);

fprintf('--- GWO-DMPC Simulation Complete ---\n\n');

% =========================================================================

% PLOT COMPARISON RESULTS

% =========================================================================

% fprintf('Plotting comparison results...\n');

% % Figure 1: DC Bus Voltage

% figure('Name', 'DC Bus Voltage Comparison');

% plot(results_dempc.t, results_dempc.X(:,5), 'b-', 'LineWidth', 1.5);

% hold on;

% plot(results_gwo.t, results_gwo.X(:,5), 'r--', 'LineWidth', 1.5);

% plot(results_dempc.t, results_dempc.Vdc_ref, 'k:', 'LineWidth', 1);

% grid on;

% title('DC Bus Voltage (V_{dc})');

% xlabel('Time (s)');

% ylabel('Voltage (V)');

% legend('DMPC (Grid Search)', 'DMPC (GWO)', 'V_{dc} Reference');

% ylim([270 330]);

% % Figure 2: Power Profiles

% figure('Name', 'Power Profile Comparison');

% subplot(4,1,1);

% plot(results_dempc.t, results_dempc.Ppv/1000, 'b-', results_gwo.t, results_gwo.Ppv/1000, 'r--');

% hold on; plot(results_dempc.t, results_dempc.Pmax/1000, 'g:');

% grid on; title('PV Power (P_{pv})'); ylabel('Power (kW)');

% legend('DMPC', 'GWO', 'P_{max}');

% subplot(4,1,2);

% plot(results_dempc.t, results_dempc.Pae/1000, 'b-', results_gwo.t, results_gwo.Pae/1000, 'r--');

% grid on; title('Electrolyzer Power (P_{ae})'); ylabel('Power (kW)');

% legend('DMPC', 'GWO');

% subplot(4,1,3);

% plot(results_dempc.t, results_dempc.Ppe/1000, 'b-', results_gwo.t, results_gwo.Ppe/1000, 'r--');

% grid on; title('Fuel Cell Power (P_{pe})'); ylabel('Power (kW)');

% legend('DMPC', 'GWO');

% subplot(4,1,4);

% plot(results_dempc.t, results_dempc.Pbat/1000, 'b-', results_gwo.t, results_gwo.Pbat/1000, 'r--');

% grid on; title('Battery Power (P_{bat})'); ylabel('Power (kW)');

% xlabel('Time (s)');

% legend('DMPC', 'GWO');

% % Figure 3: Duty Cycles

% figure('Name', 'Duty Cycle Comparison');

% subplot(4,1,1);

% plot(results_dempc.t, results_dempc.U(:,1), 'b-', results_gwo.t, results_gwo.U(:,1), 'r--');

% grid on; title('PV Duty Cycle (S_s)'); ylabel('Duty Cycle');

% legend('DMPC', 'GWO');

% subplot(4,1,2);

% plot(results_dempc.t, results_dempc.U(:,2), 'b-', results_gwo.t, results_gwo.U(:,2), 'r--');

% grid on; title('AE Duty Cycle (S_{ae})'); ylabel('Duty Cycle');

% legend('DMPC', 'GWO');

% subplot(4,1,3);

% plot(results_dempc.t, results_dempc.U(:,3), 'b-', results_gwo.t, results_gwo.U(:,3), 'r--');

% grid on; title('FC Duty Cycle (S_{pe})'); ylabel('Duty Cycle');

% legend('DMPC', 'GWO');

% subplot(4,1,4);

% plot(results_dempc.t, results_dempc.U(:,4), 'b-', results_gwo.t, results_gwo.U(:,4), 'r--');

% grid on; title('Battery Duty Cycle (S_b)'); ylabel('Duty Cycle');

% xlabel('Time (s)');

% legend('DMPC', 'GWO');

% % Figure 4: Hydrogen Tank Pressure

% figure('Name', 'Hydrogen Tank Pressure Comparison');

% plot(results_dempc.t, results_dempc.Ptank / 1e5, 'b-', 'LineWidth', 1.5);

% hold on;

% plot(results_gwo.t, results_gwo.Ptank / 1e5, 'r--', 'LineWidth', 1.5);

% grid on;

% title('Hydrogen Tank Pressure');

% xlabel('Time (s)');

% ylabel('Pressure (bar)');

% legend('DMPC (Grid Search)', 'DMPC (GWO)');

% xlim([0 t_sim]);

% % Figure 5: Battery SOC Comparison

% figure('Name', 'Battery SOC Comparison');

% plot(results_dempc.t, results_dempc.SOC * 100, 'b-', 'LineWidth', 1.5);

% hold on;

% plot(results_gwo.t, results_gwo.SOC * 100, 'r--', 'LineWidth', 1.5);

% grid on;

% title('Battery State of Charge');

% xlabel('Time (s)');

% ylabel('SOC (%)');

% legend('DMPC (Grid Search)', 'DMPC (GWO)');

% xlim([0 t_sim]);

% Calculate and display performance metrics

Vdc_IAE_dempc = sum(abs(results_dempc.X(:,5) - results_dempc.Vdc_ref)) * results_dempc.Ts;

Vdc_IAE_gwo   = sum(abs(results_gwo.X(:,5) - results_gwo.Vdc_ref)) * results_gwo.Ts;

fprintf('\n--- Performance Metrics ---\n');

fprintf('Vdc Integral Absolute Error (DMPC): %.4f\n', Vdc_IAE_dempc);

fprintf('Vdc Integral Absolute Error (GWO):  %.4f\n', Vdc_IAE_gwo);

fprintf('Total H2 Produced (DMPC): %.6f mol\n', results_dempc.total_H2_prod);

fprintf('Total H2 Produced (GWO):  %.6f mol\n', results_gwo.total_H2_prod);

fprintf('Total H2 Consumed (DMPC): %.6f mol\n', results_dempc.total_H2_cons);

fprintf('Total H2 Consumed (GWO):  %.6f mol\n', results_gwo.total_H2_cons);

fprintf('Final Tank Pressure (DMPC): %.4f bar\n', results_dempc.Ptank(end) / 1e5);

fprintf('Final Tank Pressure (GWO):  %.4f bar\n', results_gwo.Ptank(end) / 1e5);

fprintf('---------------------------\n');

================================================================================

FILE: test_DEMPC.m

================================================================================

% =========================================================================

% test_DEMPC.m

% Closed-loop DEMPC simulation of PV/Hydrogen DC Microgrid

%

% Scenario matches paper Section 4:

%   Irradiance : 1000 W/m^2, ramps to 1100 (0.1-0.2s), drops to 1000 (0.3s)

%   Load       : 30 kW, steps to 45 kW at t = 0.2s

%   Vdc setpoint: 300 V

%

% Expected behaviour:

%   t=0 to 0.2s  : Ppv > Pload → AE ON (absorbing surplus), FC OFF

%   t=0.2s+      : Ppv < Pload → AE OFF, FC ON (supplying deficit)

% =========================================================================

clc; clear; close all;

% =========================================================================

% DISTURBANCE PROFILES

% =========================================================================

% Irradiance: 1000 -> ramp to 1100 -> hold 1100 -> step to 1000 (0.2s intervals)

G_profile = @(t) 1000 .* (t < 0.2) + ...

(1000 + 100*(t-0.2)/0.2) .* (t >= 0.2 & t < 0.4) + ...

1100 .* (t >= 0.4 & t < 0.6) + ...

1000 .* (t >= 0.6);

% Temperature: fixed

T_profile = @(t) 25;

% Load: 20kW -> ramp to 40kW -> hold 40kW -> step to 50kW (0.2s intervals)

Pload_profile = @(t) 20000 .* (t < 0.2) + ...

(20000 + 20000*(t-0.2)/0.2) .* (t >= 0.2 & t < 0.4) + ...

40000 .* (t >= 0.4 & t < 0.6) + ...

50000 .* (t >= 0.6);

% =========================================================================

% INITIAL STATE

% =========================================================================

Ns_pv   = 5;    Np_pv = 20;

Voc_mod = 47.6;

G0      = G_profile(0);

T0      = T_profile(0);

Vdc_nom = 300;

% Find MPP at t=0 for initial vpv and iL

V_sw    = linspace(0.1, Ns_pv*Voc_mod, 500);

[~, Pp] = getPVArray(V_sw, G0, T0, Ns_pv, Np_pv);

[~, mi] = max(Pp);

vpv0    = V_sw(mi);

[Impp0, ~] = getPVArray(vpv0, G0, T0, Ns_pv, Np_pv);

% --- Compute initial AE current for surplus power ---

P_surplus_0 = max(vpv0*Impp0 - Pload_profile(0), 0);

iae_sw      = linspace(0, 500, 5000);

vae_sw      = zeros(size(iae_sw));

for kk = 1:length(iae_sw), [vae_sw(kk),~,~,~] = getAE(max(iae_sw(kk),1e-6)); end

[~, idx_ae] = min(abs(vae_sw .* iae_sw - P_surplus_0));

iae_init    = iae_sw(idx_ae);

ib_init  = 0;

SOC_init = 0.8; % 80% Initial SOC

x_init   = [vpv0; Impp0; iae_init; 0; Vdc_nom; ib_init; SOC_init];

fprintf('=== DEMPC Simulation Setup ===\n');

fprintf('  vpv0  = %.2f V\n',   vpv0);

fprintf('  iL0   = %.2f A\n',   Impp0);

fprintf('  Ppv0  = %.2f kW\n',  vpv0*Impp0/1000);

% =========================================================================

% RUN DEMPC  (start with short run to verify, then extend t_sim)

% =========================================================================

t_sim   = 0.8;      % Extended to 800ms for full 4-stage profile

tic;

results = runDEMPC(x_init, t_sim, G_profile, T_profile, Pload_profile);

elapsed = toc;

fprintf('Wall-clock time: %.1f s\n', elapsed);

% =========================================================================

% PLOTS

% =========================================================================

t   = results.t * 1000;     % ms

Pl  = arrayfun(Pload_profile, results.t);

G_t = arrayfun(G_profile,     results.t);

figure('Color','w','Position',[30 30 1400 900]);

sgtitle('DEMPC — Closed-Loop PV/Hydrogen DC Microgrid', ...

'FontSize', 14, 'FontWeight', 'bold');

% 1. Irradiance and load

subplot(3,3,1);

yyaxis left;

plot(t, G_t, 'b', 'LineWidth', 2); ylabel('G (W/m^2)');

yyaxis right;

plot(t, Pl/1000, 'r', 'LineWidth', 2); ylabel('P_{load} (kW)');

grid on; xlabel('Time (ms)');

title('Disturbances: Irradiance & Load');

% 2. DC bus voltage

subplot(3,3,2);

plot(t, results.X(:,5), 'k', 'LineWidth', 2); hold on;

plot(t, results.Vdc_ref, 'b--', 'LineWidth', 1.5);

yline(300, 'r:', 'V_{dc}^*=300V', 'LineWidth', 1.2);

grid on; xlabel('Time (ms)'); ylabel('V_{dc} (V)');

title('DC Bus Voltage');

ylim([270 330]);

legend('V_{dc}','V_{dc}^{ref}','Location','best');

% 3. Power balance

subplot(3,3,3);

plot(t, (results.Ppv+results.Ppe+results.Pbat-results.Pae)/1000, 'b', 'LineWidth', 2); hold on;

plot(t, Pl/1000, 'r--', 'LineWidth', 2);

grid on; xlabel('Time (ms)'); ylabel('Power (kW)');

title('Power Supply vs Demand');

legend('Supply','P_{load}','Location','best');

% 4. MPPT tracking

subplot(3,3,4);

plot(t, results.Ppv/1000, 'b', 'LineWidth', 2); hold on;

plot(t, results.Pmax/1000, 'r--', 'LineWidth', 1.5);

grid on; xlabel('Time (ms)'); ylabel('Power (kW)');

title('PV Power vs P_{max}  (MPPT)');

legend('P_{pv}','P_{max}','Location','best');

% 5. PV converter states

subplot(3,3,5);

yyaxis left;

plot(t, results.X(:,1), 'b', 'LineWidth', 2);

ylabel('v_{pv} (V)');

yyaxis right;

plot(t, results.X(:,2), 'r', 'LineWidth', 1.5);

ylabel('i_L (A)');

grid on; xlabel('Time (ms)');

title('PV Boost Converter States');

% 6. H2 and Battery Powers

subplot(3,3,6);

plot(t, results.Pae/1000, 'r',  'LineWidth', 2); hold on;

plot(t, results.Ppe/1000, 'g',  'LineWidth', 2);

plot(t, results.Pbat/1000, 'm',  'LineWidth', 2);

grid on; xlabel('Time (ms)'); ylabel('Power (kW)');

title('Storage & H_2 Powers');

legend('P_{ae} (AE)','P_{pe} (FC)', 'P_{bat}', 'Location','best');

% 7. Switching signals

subplot(3,3,7);

stairs(t, results.U(:,1), 'b',  'LineWidth', 1.5); hold on;

stairs(t, results.U(:,2), 'r',  'LineWidth', 1.5);

stairs(t, results.U(:,3), 'g',  'LineWidth', 1.5);

stairs(t, results.U(:,4), 'm',  'LineWidth', 1.5);

ylim([-0.05, 1.05]); grid on;

xlabel('Time (ms)'); ylabel('Duty Cycle');

title('Control Inputs  [d_s, d_{ae}, d_{pe}]');

legend('d_s','d_{ae}','d_{pe}','d_b','Location','best');

% 8. EMS modes

subplot(3,3,8);

stairs(t, results.zeta(:,1), 'r', 'LineWidth', 2); hold on;

stairs(t, results.zeta(:,2), 'g', 'LineWidth', 2);

ylim([-0.2, 1.4]); grid on;

xlabel('Time (ms)'); ylabel('Mode');

title('EMS Operational Modes');

legend('\zeta_a (AE on)','\zeta_p (FC on)','Location','best');

% 9. Hydrogen rates

subplot(3,3,9);

plot(t, results.NH2*1000, 'g', 'LineWidth', 2); hold on;

plot(t, results.qH2*1000, 'r', 'LineWidth', 2);

grid on; xlabel('Time (ms)'); ylabel('Rate (mmol/s)');

title('H_2 Production (AE) & Consumption (FC)');

legend('N_{H2}','q_{H2}','Location','best');

% 10. Hydrogen Tank Pressure (New Figure)

figure('Color','w','Position',[50 50 600 400]);

plot(t, results.Ptank / 1e5, 'b', 'LineWidth', 2);

grid on;

xlabel('Time (ms)');

ylabel('Pressure (bar)');

title('Hydrogen Tank Pressure');

xlim([0 t(end)]);

% figure('Color','w','Position',[30 30 1400 900]);

% sgtitle('Switching signals', ...

%         'FontSize', 14, 'FontWeight', 'bold');

% subplot(3,1,1);

% stairs(t, results.U(:,1), 'b',  'LineWidth', 1.5);

% ylim([-0.2, 1.2]); grid on;

% xlabel('Time (ms)'); ylabel('Switch Signal');

% title('Control Inputs  [S_s]');

% legend('S_s','Location','best');

%

% subplot(3,1,2);

% stairs(t, results.U(:,2), 'r',  'LineWidth', 1.5);

% ylim([-0.2, 1.2]); grid on;

% xlabel('Time (ms)'); ylabel('Switch Signal');

% title('Control Inputs  [S_{ae}]');

% legend('S_{ae}','Location','best');

%

% subplot(3,1,3);

% stairs(t, results.U(:,3), 'g',  'LineWidth', 1.5);

% ylim([-0.2, 1.2]); grid on;

% xlabel('Time (ms)'); ylabel('Switch Signal');

% title('Control Inputs  [S_{pe}]');

% legend('S_{pe}','Location','best');

% =========================================================================

% PERFORMANCE METRICS

% =========================================================================

Vdc_ss  = results.X(:,5);

Pbal    = results.Pbal;

fprintf('\n=== DEMPC Performance Metrics ===\n');

fprintf('Vdc mean          : %.3f V   (target 300 V)\n', mean(Vdc_ss));

fprintf('Vdc std dev       : %.4f V\n',                   std(Vdc_ss));

fprintf('Vdc max deviation : %.4f V\n',                   max(abs(Vdc_ss-300)));

fprintf('Power balance RMSE: %.3f kW\n',                  rms(Pbal)/1000);

fprintf('PV utilization    : %.2f %%\n', ...

100*mean(results.Ppv(results.Pmax>0)) / mean(results.Pmax(results.Pmax>0)));

fprintf('AE active fraction: %.2f %%\n', 100*mean(results.zeta(:,1)));

fprintf('FC active fraction: %.2f %%\n', 100*mean(results.zeta(:,2)));

fprintf('Total H2 produced : %.6f mol\n', results.total_H2_prod);

fprintf('Total H2 consumed : %.6f mol\n', results.total_H2_cons);

fprintf('Final Tank Press. : %.4f bar\n', results.Ptank(end) / 1e5);

================================================================================

FILE: test_GWO_DMPC.m

================================================================================

% =========================================================================

% test_GWO_DMPC.m

%

% Runs a closed-loop simulation of the DC Microgrid using the GWO-based

% Distributed Model Predictive Controller (DMPC).

%

% This script will:

% 1. Define the simulation scenario (initial state, duration, disturbances).

% 2. Run the 'runDEMPC_GWO' simulation.

% 3. Plot the key results.

% =========================================================================

% clear; clc; close all;

% ---- Simulation Scenario ----

t_sim = 0.8; % Extended to 800ms for full 4-stage profile

% ---- Disturbance Profiles (matching test_DEMPC.m) ----

% Irradiance: 1000 -> ramp to 1100 -> hold 1100 -> step to 1000 (0.2s intervals)

G_profile = @(t) 1000 .* (t < 0.2) + ...

(1000 + 100*(t-0.2)/0.2) .* (t >= 0.2 & t < 0.4) + ...

1100 .* (t >= 0.4 & t < 0.6) + ...

1000 .* (t >= 0.6);

% Temperature: fixed

T_profile = @(t) 25;

% Load: 20kW -> ramp to 40kW -> hold 40kW -> step to 50kW (0.2s intervals)

Pload_profile = @(t) 20000 .* (t < 0.2) + ...

(20000 + 20000*(t-0.2)/0.2) .* (t >= 0.2 & t < 0.4) + ...

40000 .* (t >= 0.4 & t < 0.6) + ...

50000 .* (t >= 0.6);

% ---- Initial State Calculation (matching test_DEMPC.m) ----

Ns_pv   = 5;    Np_pv = 20;

Voc_mod = 47.6;

G0      = G_profile(0);

T0      = T_profile(0);

Vdc_nom = 300;

% Find MPP at t=0 for initial vpv and iL

V_sw    = linspace(0.1, Ns_pv*Voc_mod, 500);

[~, Pp] = getPVArray(V_sw, G0, T0, Ns_pv, Np_pv);

[~, mi] = max(Pp);

vpv0    = V_sw(mi);

[Impp0, Ppv0] = getPVArray(vpv0, G0, T0, Ns_pv, Np_pv);

% Compute initial AE current for surplus power

P_surplus_0 = max(Ppv0 - Pload_profile(0), 0);

iae_sw      = linspace(0, 500, 5000);

[vae_sw, ~, ~, ~] = getAE(max(iae_sw,1e-6));

[~, idx_ae] = min(abs(vae_sw .* iae_sw - P_surplus_0));

iae_init    = iae_sw(idx_ae);

ib_init  = 0;

SOC_init = 0.8; % 80% Initial SOC

x_init   = [vpv0; Impp0; iae_init; 0; Vdc_nom; ib_init; SOC_init];

% =========================================================================

% RUN SIMULATION

% =========================================================================

% Run the new DMPC with Grey Wolf Optimizer

fprintf('--- Starting GWO-DMPC Simulation ---\n');

tic;

results_gwo = runDEMPC_GWO(x_init, t_sim, G_profile, T_profile, Pload_profile);

elapsed_time = toc;

fprintf('--- GWO-DMPC Simulation Complete (%.2f seconds) ---\n\n', elapsed_time);

% =========================================================================

% PLOT RESULTS

% =========================================================================

fprintf('Plotting GWO-DMPC results...\n');

% Figure 1: DC Bus Voltage

figure('Name', 'GWO-DMPC: DC Bus Voltage');

plot(results_gwo.t, results_gwo.X(:,5), 'b-', 'LineWidth', 1.5);

hold on;

plot(results_gwo.t, results_gwo.Vdc_ref, 'r--', 'LineWidth', 1.5);

grid on;

title('DC Bus Voltage (V_{dc})');

xlabel('Time (s)');

ylabel('Voltage (V)');

legend('V_{dc} (GWO)', 'V_{dc} Reference');

ylim([270 330]);

% Figure 2: Power Profiles

figure('Name', 'GWO-DMPC: Power Profiles');

subplot(4,1,1);

plot(results_gwo.t, results_gwo.Ppv/1000, 'b-', 'LineWidth', 1.5);

hold on; plot(results_gwo.t, results_gwo.Pmax/1000, 'r--', 'LineWidth', 1.5);

grid on; title('PV Power (P_{pv})'); ylabel('Power (kW)');

legend('P_{pv} (GWO)', 'P_{max}');

subplot(4,1,2);

plot(results_gwo.t, results_gwo.Pae/1000, 'b-', 'LineWidth', 1.5);

grid on; title('Electrolyzer Power (P_{ae})'); ylabel('Power (kW)');

legend('P_{ae} (GWO)');

subplot(4,1,3);

plot(results_gwo.t, results_gwo.Ppe/1000, 'b-', 'LineWidth', 1.5);

grid on; title('Fuel Cell Power (P_{pe})'); ylabel('Power (kW)');

legend('P_{pe} (GWO)');

subplot(4,1,4);

plot(results_gwo.t, results_gwo.Pbat/1000, 'm-', 'LineWidth', 1.5);

grid on; title('Battery Power (P_{bat})'); ylabel('Power (kW)');

xlabel('Time (s)');

legend('P_{bat} (GWO)');

% Figure 3: Duty Cycles

figure('Name', 'GWO-DMPC: Duty Cycles');

subplot(4,1,1);

stairs(results_gwo.t, results_gwo.U(:,1), 'b-', 'LineWidth', 1.5);

grid on; title('PV Duty Cycle (S_s)'); ylabel('Duty Cycle');

legend('d_s (GWO)');

subplot(4,1,2);

stairs(results_gwo.t, results_gwo.U(:,2), 'b-', 'LineWidth', 1.5);

grid on; title('AE Duty Cycle (S_{ae})'); ylabel('Duty Cycle');

legend('d_{ae} (GWO)');

subplot(4,1,3);

stairs(results_gwo.t, results_gwo.U(:,3), 'b-', 'LineWidth', 1.5);

grid on; title('FC Duty Cycle (S_{pe})'); ylabel('Duty Cycle');

legend('d_{pe} (GWO)');

subplot(4,1,4);

stairs(results_gwo.t, results_gwo.U(:,4), 'm-', 'LineWidth', 1.5);

grid on; title('Battery Duty Cycle (S_b)'); ylabel('Duty Cycle');

xlabel('Time (s)');

legend('d_b (GWO)');

% Figure 4: Hydrogen Tank Pressure

figure('Name', 'GWO-DMPC: Hydrogen Tank Pressure');

plot(results_gwo.t, results_gwo.Ptank / 1e5, 'b-', 'LineWidth', 1.5);

grid on;

title('Hydrogen Tank Pressure');

xlabel('Time (s)');

ylabel('Pressure (bar)');

xlim([0 results_gwo.t(end)]);

% Figure 5: Battery SOC

figure('Name', 'GWO-DMPC: Battery SOC');

plot(results_gwo.t, results_gwo.SOC * 100, 'm-', 'LineWidth', 1.5);

grid on;

title('Battery State of Charge');

xlabel('Time (s)');

ylabel('SOC (%)');

xlim([0 results_gwo.t(end)]);

% Calculate and display performance metrics

Vdc_IAE_gwo   = sum(abs(results_gwo.X(:,5) - results_gwo.Vdc_ref)) * results_gwo.Ts;

fprintf('\n--- GWO-DMPC Performance Metrics ---\n');

fprintf('Simulation Wall-Clock Time:       %.2f seconds\n', elapsed_time);

fprintf('Vdc Integral Absolute Error (IAE): %.4f\n', Vdc_IAE_gwo);

fprintf('Total H2 Produced:                 %.6f mol\n', results_gwo.total_H2_prod);

fprintf('Total H2 Consumed:                 %.6f mol\n', results_gwo.total_H2_cons);

fprintf('Final Tank Pressure:               %.4f bar\n', results_gwo.Ptank(end) / 1e5);

fprintf('-------------------------------------\n');

================================================================================

FILE: test_RL_Environment.m

================================================================================

% =========================================================================

% test_RL_Environment.m

% Demonstrates the RL Macro-Model running a 10-minute simulation instantly.

% =========================================================================

clear; clc; close all;

% Simulation settings

dt = 1;              % 1-second time step

t_end = 600;         % 10 minutes

N_steps = t_end / dt;

% Initial State

SOC0   = 0.50;       % 50% SOC

Ptank0 = 10 * 1e5;   % 10 bar

state  = [SOC0; Ptank0];

% Logging arrays

log_SOC = zeros(N_steps, 1);

log_Ptnk = zeros(N_steps, 1);

log_Rwd = zeros(N_steps, 1);

log_Act = zeros(N_steps, 1);

fprintf('Running 10-Minute RL Environment Test...\n');

tic;

prev_action = 1;

for k = 1:N_steps

% Simulated Disturbances (Randomized step load and PV)

Ppv   = 35000 + 5000 * sin(2*pi*k/300); % PV waving between 30kW and 40kW

Pload = 30000 + 15000 * (mod(k, 120) > 60); % Load stepping between 30kW and 45kW

% THE AGENT'S ACTION (Random for now, later replaced by Neural Network)

% 1 = Idle, 2 = AE ON, 3 = FC ON

action = randi([1, 3]);

% Step the Environment

[next_state, reward, is_done, info] = stepMicrogridEnv(state, action, prev_action, Ppv, Pload, dt);

% Log

log_SOC(k) = state(1);

log_Ptnk(k)= state(2);

log_Rwd(k) = reward;

log_Act(k) = action;

% Transition

state = next_state;

prev_action = action;

if is_done

fprintf('Episode terminated early at step %d due to fatal SOC limits!\n', k);

break;

end

end

fprintf('Simulation completed in %.4f seconds.\n', toc);

% Plotting the Agent's Experience

figure('Color', 'w', 'Position', [100 100 800 600]);

subplot(3,1,1);

plot(log_SOC * 100, 'm', 'LineWidth', 1.5); title('Battery SOC (%)'); grid on;

subplot(3,1,2);

plot(log_Ptnk / 1e5, 'b', 'LineWidth', 1.5); title('Tank Pressure (Bar)'); grid on;

subplot(3,1,3);

stairs(log_Act, 'k', 'LineWidth', 1.5); title('Agent Actions (1=Idle, 2=AE, 3=FC)');

yticks([1 2 3]); ylim([0.5 3.5]); grid on;

xlabel('Time (seconds)');

================================================================================

FILE: test_Trained_QLearning.m

================================================================================

% =========================================================================

% test_Trained_QLearning.m

% Evaluates the fully trained Q-Learning Agent on the Macro Environment.

% =========================================================================

clear; clc; close all;

% Load the trained brain (Q-Table)

if ~isfile('Q_Table_EMS.mat')

error('Could not find Q_Table_EMS.mat. Please run train_QLearning_EMS.m first.');

end

load('Q_Table_EMS.mat', 'Q');

% Simulation settings

dt = 1;              % 1-second time step

t_end = 600;         % 10 minutes

N_steps = t_end / dt;

% Initial State

SOC0   = 0.50;       % 50% SOC

Ptank0 = 10 * 1e5;   % 10 bar

state  = [SOC0; Ptank0];

% Logging arrays

log_SOC  = zeros(N_steps, 1);

log_Ptnk = zeros(N_steps, 1);

log_Act  = zeros(N_steps, 1);

log_Pnet = zeros(N_steps, 1);

fprintf('Running 10-Minute Evaluation with Trained AI...\n');

prev_action = 1;

for k = 1:N_steps

% Simulated Disturbances (Same as training)

Ppv   = 35000 + 5000 * sin(2*pi*k/300);

Pload = 30000 + 15000 * (mod(k, 120) > 60);

Pnet  = Ppv - Pload;

% 1. Convert raw readings into Discrete States (Buckets)

SOC = state(1);

if SOC < 0.30; s_soc = 1;

elseif SOC <= 0.80; s_soc = 2;

else; s_soc = 3; end

if Pnet < -5000; s_pnet = 1;

elseif Pnet <= 5000; s_pnet = 2;

else; s_pnet = 3; end

% 2. THE AI'S ACTION (100% Exploit, 0% Random)

[~, action] = max(Q(s_soc, s_pnet, :));

% Step the Environment

[next_state, ~, is_done, ~] = stepMicrogridEnv(state, action, prev_action, Ppv, Pload, dt);

% Log

log_SOC(k)  = state(1);

log_Ptnk(k) = state(2);

log_Act(k)  = action;

log_Pnet(k) = Pnet;

% Transition

state = next_state;

prev_action = action;

if is_done

fprintf('Episode terminated early at step %d due to fatal SOC limits!\n', k);

break;

end

end

% Plotting the Agent's Performance

figure('Color', 'w', 'Position', [100 100 800 800]);

subplot(4,1,1); plot(log_Pnet / 1000, 'k', 'LineWidth', 1.5); title('Net Power (PV - Load) [kW]'); grid on;

subplot(4,1,2); plot(log_SOC * 100, 'm', 'LineWidth', 1.5); title('Battery SOC (%)'); grid on;

subplot(4,1,3); plot(log_Ptnk / 1e5, 'b', 'LineWidth', 1.5); title('Tank Pressure (Bar)'); grid on;

subplot(4,1,4); stairs(log_Act, 'r', 'LineWidth', 2); title('AI Actions (1=Idle, 2=AE, 3=FC)');

yticks([1 2 3]); ylim([0.5 3.5]); grid on;

xlabel('Time (seconds)');

================================================================================

FILE: train_QLearning_EMS.m

================================================================================

% =========================================================================

% train_QLearning_EMS.m

% Trains a Tabular Q-Learning Agent to replace the rule-based EMS.

% =========================================================================

clear; clc; close all;

% --- RL Hyperparameters ---

num_episodes = 500;       % Number of 10-minute training runs

alpha        = 0.1;       % Learning rate (how fast it updates memory)

gamma        = 0.95;      % Discount factor (how much it cares about the future)

epsilon      = 1.0;       % Exploration rate (100% random at start)

epsilon_min  = 0.05;      % Minimum exploration rate

epsilon_decay= 0.99;      % Decay rate per episode

% --- Define the Discrete State Space ---

% To use a Q-Table, we must group continuous numbers into simple buckets.

% SOC States: 1 = Low (<30%), 2 = Normal (30-80%), 3 = High (>80%)

% Net Power States: 1 = Deficit (<-5kW), 2 = Balanced (±5kW), 3 = Surplus (>5kW)

num_soc_states  = 3;

num_pnet_states = 3;

num_actions     = 3; % 1=Idle, 2=AE, 3=FC

% Initialize the Q-Table (Cheat Sheet) with zeros

Q = zeros(num_soc_states, num_pnet_states, num_actions);

% Logging

reward_history = zeros(num_episodes, 1);

fprintf('Starting Q-Learning Training for %d episodes...\n', num_episodes);

tic;

for episode = 1:num_episodes

% Reset Environment for a new 10-minute run

dt = 1; N_steps = 600;

% Randomize initial SOC between 15% and 95% so the AI explores all states!

state = [0.15 + 0.80 * rand(); 10e5];

prev_action = 1;      % Start Idle

total_reward = 0;

for k = 1:N_steps

% 1. Get Current Disturbances

Ppv   = 35000 + 5000 * sin(2*pi*k/300);

Pload = 30000 + 15000 * (mod(k, 120) > 60);

Pnet  = Ppv - Pload;

% 2. Convert raw readings into Discrete States (Buckets)

SOC = state(1);

if SOC < 0.30; s_soc = 1;

elseif SOC <= 0.80; s_soc = 2;

else; s_soc = 3; end

if Pnet < -5000; s_pnet = 1;

elseif Pnet <= 5000; s_pnet = 2;

else; s_pnet = 3; end

% 3. Choose Action (Epsilon-Greedy)

if rand() < epsilon

% Explore: Pick a random action

action = randi([1, 3]);

else

% Exploit: Pick the best action from the Q-Table

[~, action] = max(Q(s_soc, s_pnet, :));

end

% 4. Step the Environment

[next_state, reward, is_done, ~] = stepMicrogridEnv(state, action, prev_action, Ppv, Pload, dt);

% 5. Get the Next Discrete State

next_SOC = next_state(1);

if next_SOC < 0.30; next_s_soc = 1;

elseif next_SOC <= 0.80; next_s_soc = 2;

else; next_s_soc = 3; end

% (Pnet for the next step is unknown to the agent, so we assume it stays roughly the same for the Q-update)

next_s_pnet = s_pnet;

% 6. Update the Q-Table (The Core Learning Equation)

best_future_Q = max(Q(next_s_soc, next_s_pnet, :));

current_Q = Q(s_soc, s_pnet, action);

Q(s_soc, s_pnet, action) = current_Q + alpha * (reward + gamma * best_future_Q - current_Q);

% 7. Transition to next step

state = next_state;

prev_action = action;

total_reward = total_reward + reward;

if is_done

break; % Grid crashed, end episode early

end

end

% Decay exploration rate so it acts smarter over time

epsilon = max(epsilon_min, epsilon * epsilon_decay);

reward_history(episode) = total_reward;

% Print progress

if mod(episode, 50) == 0

fprintf('Episode %3d | Total Reward: %6.1f | Epsilon: %.2f\n', episode, total_reward, epsilon);

end

end

fprintf('Training complete in %.2f seconds.\n', toc);

% Plot Learning Curve

figure('Color','w');

plot(1:num_episodes, reward_history, 'b', 'LineWidth', 1.5);

xlabel('Training Episode'); ylabel('Total Reward');

title('AI Learning Curve (Higher is Better)');

grid on;

% Save the trained AI brain

save('Q_Table_EMS.mat', 'Q');

================================================================================

FILE: visualizations.m

================================================================================

% =========================================================================

% visualizations.m

% Generates publication-quality figures comparing FCS-DEMPC and CCS-DEMPC.

%

% Assumes the workspace contains:

%   - results_dempc (from FCS-DEMPC simulation)

%   - results_gwo   (from CCS-DEMPC simulation)

% =========================================================================

% Verify that data is loaded

if ~exist('results_dempc', 'var') || ~exist('results_gwo', 'var')

error('Results not found in workspace. Please run test_comparison.m or the individual simulations first.');

end

close all;

clc;

t = results_dempc.t;

set(0,'DefaultFigureColor','w')

% Recreate disturbance profiles for plotting

G_profile = @(t) 1000 .* (t < 0.2) + ...

(1000 + 100*(t-0.2)/0.2) .* (t >= 0.2 & t < 0.4) + ...

1100 .* (t >= 0.4 & t < 0.6) + ...

1000 .* (t >= 0.6);

Pload_profile = @(t) 20000 .* (t < 0.2) + ...

(20000 + 20000*(t-0.2)/0.2) .* (t >= 0.2 & t < 0.4) + ...

40000 .* (t >= 0.4 & t < 0.6) + ...

50000 .* (t >= 0.6);

G_t     = arrayfun(G_profile, t);

Pload_t = arrayfun(Pload_profile, t) / 1000; % kW

Pmax_t  = results_dempc.Pmax / 1000;         % kW

Pnet_t  = Pmax_t - Pload_t;                  % kW

stages = [0.2, 0.4, 0.6]; % Stage boundaries

% =========================================================================

% FIGURE 1: Simulation Scenario and EMS Response

% =========================================================================

f1 = figure('Name', 'Figure 1: Simulation Scenario and EMS Response', 'Position', [100, 100, 800, 700], 'Color', 'w');

% Top Panel: Irradiance and Load

ax1_1 = subplot(3, 1, 1);

yyaxis left;

plot(t, G_t, 'b-', 'LineWidth', 2);

ylabel('Irradiance G (W/m^2)');

ylim([900 1200]);

yyaxis right;

plot(t, Pload_t, 'r-', 'LineWidth', 2);

ylabel('Load P_{load} (kW)');

ylim([15 55]);

xline(stages, 'k--');

text(0.1, 52, 'Stage 1', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

text(0.3, 52, 'Stage 2', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

text(0.5, 52, 'Stage 3', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

text(0.7, 52, 'Stage 4', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

%title('Disturbance Profiles');

% text(ax1_1, 0.5, -0.2, '(a) Irradiance and Load profile w.r.t. time', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

text(ax1_1, 0.5, -0.2, '(a)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

grid on;

xlim([0, 0.8]);

% Middle Panel: EMS Activation Flags

ax1_2 =subplot(3, 1, 2);

stairs(t, results_dempc.zeta(:, 1), 'b-', 'LineWidth', 2); hold on;

stairs(t, results_dempc.zeta(:, 2), 'r--', 'LineWidth', 2);

xline(stages, 'k--');

ylabel('EMS Flags');

yticks([0 1]);

ylim([-0.2 1.4]);

legend('\zeta_a (Electrolyzer)', '\zeta_p (Fuel Cell)', 'Location', 'best');

%title('EMS Activation Response');

% text(ax1_2, 0.5, -0.15, '(b) EMS Activation Response', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

text(ax1_2, 0.5, -0.25, '(b)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

grid on;

xlim([0, 0.8]);

% Bottom Panel: Net Power (Surplus/Deficit)

ax1_3 = subplot(3, 1, 3);

plot(t, Pnet_t, 'k-', 'LineWidth', 1.5); hold on;

yline(0, 'g-', 'LineWidth', 1.5);

xline(stages, 'k--');

xlabel('Time (s)');

ylabel('Net Power P_{net} (kW)');

%title('Power Surplus and Deficit (P_{pv,max} - P_{load})');

% text(ax1_3, 0.5, -0.35, '(c) Net Power (Surplus/Deficit)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

text(ax1_3, 0.5, -0.37, '(c)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

grid on;

xlim([0, 0.8]);

% =========================================================================

% FIGURE 2: DC Bus Voltage Regulation (Full View + Two Zooms)

% =========================================================================

f2 = figure('Name', 'Figure 2: DC Bus Voltage Regulation', 'Position', [150, 150, 900, 500], 'Color', 'w');

ax_main2 = axes('Position', [0.1, 0.1, 0.85, 0.85]);

plot(ax_main2, t, results_dempc.X(:,5), 'b-', 'LineWidth', 1.5); hold on;

plot(ax_main2, t, results_gwo.X(:,5), 'r--', 'LineWidth', 1.5);

plot(ax_main2, t, results_dempc.Vdc_ref, 'k:', 'LineWidth', 1.5);

xline(ax_main2, stages, 'k--');

xlabel('Time (s)');

ylabel('DC Bus Voltage (V)');

legend('DMPC', 'Proposed GWO-DMPC', 'V_{dc,ref}', 'Location', 'northwest');

% text(ax_main2, 0.5, -0.15, 'DC Bus Voltage regulation', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

grid on;

xlim([0, 0.8]);

ylim([285, 315]);

% --- Zoom 1 (t = 0.2 s) ---

xlim1 = [0.185, 0.210];

idx1 = (t >= xlim1(1) & t <= xlim1(2));

ymin1 = min([min(results_dempc.X(idx1,5)), min(results_gwo.X(idx1,5)), min(results_dempc.Vdc_ref(idx1))]);

ymax1 = max([max(results_dempc.X(idx1,5)), max(results_gwo.X(idx1,5)), max(results_dempc.Vdc_ref(idx1))]);

margin1 = max(0.5, (ymax1 - ymin1) * 0.2);

% Draw bounding box on main axes

rectangle(ax_main2, 'Position', [xlim1(1), ymin1-margin1, xlim1(2)-xlim1(1), (ymax1+margin1)-(ymin1-margin1)], 'EdgeColor', 'k', 'LineStyle', '-.');

% Create Inset Axes

ax_ins1 = axes('Position', [0.22, 0.2, 0.2, 0.25]); % Bottom-leftish

box on; grid on; hold on;

plot(ax_ins1, t, results_dempc.X(:,5), 'b-', 'LineWidth', 1.5);

plot(ax_ins1, t, results_gwo.X(:,5), 'r--', 'LineWidth', 1.5);

plot(ax_ins1, t, results_dempc.Vdc_ref, 'k:', 'LineWidth', 1.5);

xlim(ax_ins1, xlim1);

ylim(ax_ins1, [ymin1-margin1, ymax1+margin1]);

%title(ax_ins1, 'Zoom: 0.18s - 0.28s', 'FontSize', 9);

% --- Zoom 2 (t = 0.6 s) ---

xlim2 = [0.56, 0.65];

idx2 = (t >= xlim2(1) & t <= xlim2(2));

ymin2 = min([min(results_dempc.X(idx2,5)), min(results_gwo.X(idx2,5)), min(results_dempc.Vdc_ref(idx2))]);

ymax2 = max([max(results_dempc.X(idx2,5)), max(results_gwo.X(idx2,5)), max(results_dempc.Vdc_ref(idx2))]);

margin2 = max(0.5, (ymax2 - ymin2) * 0.2);

% Draw bounding box on main axes

rectangle(ax_main2, 'Position', [xlim2(1), ymin2-margin2, xlim2(2)-xlim2(1), (ymax2+margin2)-(ymin2-margin2)], 'EdgeColor', 'k', 'LineStyle', '-.');

% Create Inset Axes

ax_ins2 = axes('Position', [0.65, 0.2, 0.2, 0.25]); % Bottom-rightish

box on; grid on; hold on;

plot(ax_ins2, t, results_dempc.X(:,5), 'b-', 'LineWidth', 1.5);

plot(ax_ins2, t, results_gwo.X(:,5), 'r--', 'LineWidth', 1.5);

plot(ax_ins2, t, results_dempc.Vdc_ref, 'k:', 'LineWidth', 1.5);

xlim(ax_ins2, xlim2);

ylim(ax_ins2, [ymin2-margin2, ymax2+margin2]);

%title(ax_ins2, 'Zoom: 0.58s - 0.68s', 'FontSize', 9);

% =========================================================================

% FIGURE 3: Duty Cycle Trajectories

% =========================================================================

f3 = figure('Name', 'Figure 3: Duty Cycle Trajectories', 'Position', [200, 100, 800, 800], 'Color', 'w');

% PV Duty Cycle

ax3_1 = subplot(2, 2, 1);

stairs(t, results_dempc.U(:,1), 'b-', 'LineWidth', 1.5); hold on;

plot(t, results_gwo.U(:,1), 'r--', 'LineWidth', 1.5);

ylabel('d_s');

%title('PV Duty Cycle');

% text(ax3_1, 0.5, -0.15, '(a). PV Duty Cycle', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

text(ax3_1, 0.5, -0.15, '(a)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

grid on; xlim([0, 0.8]); ylim([-0.1 1.1]);

% lgd1 =legend('DMPC', 'Proposed GWO-DMPC', 'Location', 'northwest');

% lgd1.FontSize = 6;

lgd = legend(ax3_1, 'DMPC', 'Proposed GWO-DMPC', 'P_{max}');

set(lgd, 'Position', [0.18, 0.85, 0.1, 0.05]); % [left, bottom, width, height]

lgd.FontSize = 6;

%lgd1.ItemTokenSize = [10, 8];

%legend boxoff;

% AE Duty Cycle

ax3_2 = subplot(2, 2, 2);

stairs(t, results_dempc.U(:,2), 'b-', 'LineWidth', 1.5); hold on;

plot(t, results_gwo.U(:,2), 'r--', 'LineWidth', 1.5);

ylabel('d_{ae}');

lgd = legend(ax3_2, 'DMPC', 'Proposed GWO-DMPC', 'P_{max}');

set(lgd, 'Position', [0.62, 0.85, 0.1, 0.05]); % [left, bottom, width, height]

lgd.FontSize = 6;

% lgd.ItemTokenSize = [8, 8];

%title('Electrolyzer Duty Cycle');

% text(ax3_2, 0.5, -0.15, '(b). Electrolyzer Duty Cycle', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

text(ax3_2, 0.5, -0.15, '(b)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

grid on; xlim([0, 0.8]); ylim([-0.1 1.1]);

% FC Duty Cycle

ax3_3 = subplot(2, 2, 3);

stairs(t, results_dempc.U(:,3), 'b-', 'LineWidth', 1.5); hold on;

plot(t, results_gwo.U(:,3), 'r--', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('d_{pe}');

lgd = legend(ax3_3, 'DMPC', 'Proposed GWO-DMPC', 'P_{max}');

set(lgd, 'Position', [0.22, 0.15, 0.1, 0.05]); % [left, bottom, width, height]

lgd.FontSize = 6;

%title('Fuel Cell Duty Cycle');

% text(ax3_3, 0.5, -0.15, '(c). Fuel Cell Duty Cycle', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

text(ax3_3, 0.5, -0.20, '(c)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

grid on; xlim([0, 0.8]); ylim([-0.1 1.1]);

% Battery Duty Cycle

ax3_4 = subplot(2, 2, 4);

stairs(t, results_dempc.U(:,4), 'b-', 'LineWidth', 1.5); hold on;

plot(t, results_gwo.U(:,4), 'r--', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('d_b');

lgd = legend(ax3_4, 'DMPC', 'Proposed GWO-DMPC', 'P_{max}');

set(lgd, 'Position', [0.62, 0.115, 0.1, 0.04]); % [left, bottom, width, height]

lgd.FontSize = 6;

%title('Battery Duty Cycle');

% text(ax3_4, 0.5, -0.15, '(d). Battery Duty Cycle', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

text(ax3_4, 0.5, -0.20, '(d)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold')

grid on; xlim([0, 0.8]); ylim([-0.1 1.1]);

% --- Zoom on d_s ---

xlim_ds = [0.19, 0.23];

idx_ds = (t >= xlim_ds(1) & t <= xlim_ds(2));

ymin_ds = min([min(results_dempc.U(idx_ds,1)), min(results_gwo.U(idx_ds,1))]);

ymax_ds = max([max(results_dempc.U(idx_ds,1)), max(results_gwo.U(idx_ds,1))]);

margin_ds = max(0.05, (ymax_ds - ymin_ds) * 0.1);

rectangle(ax3_1, 'Position', [xlim_ds(1), ymin_ds-margin_ds, xlim_ds(2)-xlim_ds(1), (ymax_ds+margin_ds)-(ymin_ds-margin_ds)], 'EdgeColor', 'k', 'LineStyle', '-.');

pos1 = get(ax3_1, 'Position');

ax_ins3_1 = axes('Position', [pos1(1)+pos1(3)*0.65, pos1(2)+pos1(4)*0.55, pos1(3)*0.3, pos1(4)*0.35]);

box on; grid on; hold on;

stairs(ax_ins3_1, t, results_dempc.U(:,1), 'b-', 'LineWidth', 1.5);

plot(ax_ins3_1, t, results_gwo.U(:,1), 'r--', 'LineWidth', 1.5);

xlim(ax_ins3_1, xlim_ds);

ylim(ax_ins3_1, [ymin_ds-margin_ds, ymax_ds+margin_ds]);

%title(ax_ins3_1, 'Zoom: 0.19 - 0.23 s', 'FontSize', 8);

% --- Zoom on d_ae ---

xlim_dae = [0.19, 0.23];

idx_dae = (t >= xlim_dae(1) & t <= xlim_dae(2));

ymin_dae = min([min(results_dempc.U(idx_dae,2)), min(results_gwo.U(idx_dae,2))]);

ymax_dae = max([max(results_dempc.U(idx_dae,2)), max(results_gwo.U(idx_dae,2))]);

margin_dae = max(0.05, (ymax_dae - ymin_dae) * 0.1);

rectangle(ax3_2, 'Position', [xlim_dae(1), ymin_dae-margin_dae, xlim_dae(2)-xlim_dae(1), (ymax_dae+margin_dae)-(ymin_dae-margin_dae)], 'EdgeColor', 'k', 'LineStyle', '-.');

pos2 = get(ax3_2, 'Position');

ax_ins3_2 = axes('Position', [pos2(1)+pos2(3)*0.65, pos2(2)+pos2(4)*0.55, pos2(3)*0.3, pos2(4)*0.35]);

box on; grid on; hold on;

stairs(ax_ins3_2, t, results_dempc.U(:,2), 'b-', 'LineWidth', 1.5);

plot(ax_ins3_2, t, results_gwo.U(:,2), 'r--', 'LineWidth', 1.5);

xlim(ax_ins3_2, xlim_dae);

ylim(ax_ins3_2, [ymin_dae-margin_dae, ymax_dae+margin_dae]);

%title(ax_ins3_2, 'Zoom: 0.19 - 0.23 s', 'FontSize', 8);

% --- Zoom on d_pe ---

xlim_dpe = [0.58, 0.65];

idx_dpe = (t >= xlim_dpe(1) & t <= xlim_dpe(2));

ymin_dpe = min([min(results_dempc.U(idx_dpe,3)), min(results_gwo.U(idx_dpe,3))]);

ymax_dpe = max([max(results_dempc.U(idx_dpe,3)), max(results_gwo.U(idx_dpe,3))]);

margin_dpe = max(0.05, (ymax_dpe - ymin_dpe) * 0.1);

rectangle(ax3_3, 'Position', [xlim_dpe(1), ymin_dpe-margin_dpe, xlim_dpe(2)-xlim_dpe(1), (ymax_dpe+margin_dpe)-(ymin_dpe-margin_dpe)], 'EdgeColor', 'k', 'LineStyle', '-.');

pos3 = get(ax3_3, 'Position');

ax_ins3_3 = axes('Position', [pos3(1)+pos3(3)*0.1, pos3(2)+pos3(4)*0.55, pos3(3)*0.3, pos3(4)*0.35]);

box on; grid on; hold on;

stairs(ax_ins3_3, t, results_dempc.U(:,3), 'b-', 'LineWidth', 1.5);

plot(ax_ins3_3, t, results_gwo.U(:,3), 'r--', 'LineWidth', 1.5);

xlim(ax_ins3_3, xlim_dpe);

ylim(ax_ins3_3, [ymin_dpe-margin_dpe, ymax_dpe+margin_dpe]);

%title(ax_ins3_3, 'Zoom: 0.58 - 0.65 s', 'FontSize', 8);

% --- Zoom on d_b ---

xlim_db = [0.58, 0.65];

idx_db = (t >= xlim_db(1) & t <= xlim_db(2));

ymin_db = min([min(results_dempc.U(idx_db,4)), min(results_gwo.U(idx_db,4))]);

ymax_db = max([max(results_dempc.U(idx_db,4)), max(results_gwo.U(idx_db,4))]);

margin_db = max(0.05, (ymax_db - ymin_db) * 0.1);

rectangle(ax3_4, 'Position', [xlim_db(1), ymin_db-margin_db, xlim_db(2)-xlim_db(1), (ymax_db+margin_db)-(ymin_db-margin_db)], 'EdgeColor', 'k', 'LineStyle', '-.');

pos4 = get(ax3_4, 'Position');

ax_ins3_4 = axes('Position', [pos4(1)+pos4(3)*0.1, pos4(2)+pos4(4)*0.55, pos4(3)*0.3, pos4(4)*0.35]);

box on; grid on; hold on;

stairs(ax_ins3_4, t, results_dempc.U(:,4), 'b-', 'LineWidth', 1.5);

plot(ax_ins3_4, t, results_gwo.U(:,4), 'r--', 'LineWidth', 1.5);

xlim(ax_ins3_4, xlim_db);

ylim(ax_ins3_4, [ymin_db-margin_db, ymax_db+margin_db]);

%title(ax_ins3_4, 'Zoom: 0.58 - 0.65 s', 'FontSize', 8);

% =========================================================================

% FIGURE 4: Power Dispatch Comparison

% =========================================================================

f4 = figure('Name', 'Figure 4: Power Dispatch Comparison', 'Position', [250, 100, 800, 800], 'Color', 'w');

% PV Power

ax4_1 = subplot(2, 2, 1);

plot(t, results_dempc.Ppv / 1000, 'b-', 'LineWidth', 1.5); hold on;

plot(t, results_gwo.Ppv / 1000, 'r--', 'LineWidth', 1.5);

plot(t, Pmax_t, 'k:', 'LineWidth', 1.5);

ylabel('P_{pv} (kW)');

% lgd = legend('DMPC', 'Proposed GWO-DMPC', 'P_{max}', 'Location', 'NorthWest');

% lgd.FontSize = 5;

% lgd.ItemTokenSize = [8, 8];

% legend boxoff;

lgd = legend(ax4_1, 'DMPC', 'Proposed GWO-DMPC', 'P_{max}');

set(lgd, 'Position', [0.15, 0.85, 0.1, 0.05]); % [left, bottom, width, height]

lgd.FontSize = 5;

lgd.ItemTokenSize = [8, 8];

%title('PV Power and MPPT');

% text(ax4_1, 0.5, -0.15, '(a). PV Power and MPPT', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

text(ax4_1, 0.5, -0.15, '(a)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

grid on; xlim([0, 0.8]);

% --- Zoom on P_pv ---

xlim_ppv = [0.19, 0.23];

idx_ppv = (t >= xlim_ppv(1) & t <= xlim_ppv(2));

ymin_pv = min([min(results_dempc.Ppv(idx_ppv))/1000, min(results_gwo.Ppv(idx_ppv))/1000, min(Pmax_t(idx_ppv))]);

ymax_pv = max([max(results_dempc.Ppv(idx_ppv))/1000, max(results_gwo.Ppv(idx_ppv))/1000, max(Pmax_t(idx_ppv))]);

margin_pv = max(0.5, (ymax_pv - ymin_pv) * 0.1);

rectangle(ax4_1, 'Position', [xlim_ppv(1), ymin_pv-margin_pv, xlim_ppv(2)-xlim_ppv(1), (ymax_pv+margin_pv)-(ymin_pv-margin_pv)], 'EdgeColor', 'k', 'LineStyle', '-.');

pos_ppv = get(ax4_1, 'Position');

ax_ins4_1 = axes('Position', [pos_ppv(1)+pos_ppv(3)*0.35, pos_ppv(2)+pos_ppv(4)*0.05, pos_ppv(3)*0.32, pos_ppv(4)*0.28]);

box on; grid on; hold on;

plot(ax_ins4_1, t, results_dempc.Ppv / 1000, 'b-', 'LineWidth', 1.5);

plot(ax_ins4_1, t, results_gwo.Ppv / 1000, 'r--', 'LineWidth', 1.5);

plot(ax_ins4_1, t, Pmax_t, 'k:', 'LineWidth', 1.5);

xlim(ax_ins4_1, xlim_ppv);

ylim(ax_ins4_1, [ymin_pv - margin_pv, ymax_pv + margin_pv]);

%title(ax_ins4_1, 'Zoom: 0.19 - 0.23 s', 'FontSize', 8);

% AE Power

ax4_2 = subplot(2, 2, 2);

plot(t, results_dempc.Pae / 1000, 'b-', 'LineWidth', 1.5); hold on;

plot(t, results_gwo.Pae / 1000, 'r--', 'LineWidth', 1.5);

ylabel('P_{ae} (kW)');

lgd = legend('DMPC', 'Proposed GWO-DMPC', 'P_{max}', 'Location', 'NorthWest');

lgd.FontSize = 6;

lgd.ItemTokenSize = [10, 8];

% legend boxoff;

%title('Electrolyzer Power');

% text(ax4_2, 0.5, -0.15, '(b). Electrolyzer Power', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

text(ax4_2, 0.5, -0.15, '(b)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

grid on; xlim([0, 0.8]);

% --- Zoom on P_ae ---

xlim_pae = [0.19, 0.21];

idx_pae = (t >= xlim_pae(1) & t <= xlim_pae(2));

ymin_pae = min([min(results_dempc.Pae(idx_pae)), min(results_gwo.Pae(idx_pae))]) / 1000;

ymax_pae = max([max(results_dempc.Pae(idx_pae)), max(results_gwo.Pae(idx_pae))]) / 1000;

margin_pae = max(0.5, (ymax_pae - ymin_pae) * 0.1);

rectangle(ax4_2, 'Position', [xlim_pae(1), ymin_pae-margin_pae, xlim_pae(2)-xlim_pae(1), (ymax_pae+margin_pae)-(ymin_pae-margin_pae)], 'EdgeColor', 'k', 'LineStyle', '-.');

pos_pae = get(ax4_2, 'Position');

ax_ins4_2 = axes('Position', [pos_pae(1)+pos_pae(3)*0.55, pos_pae(2)+pos_pae(4)*0.55, pos_pae(3)*0.4, pos_pae(4)*0.4]);

box on; grid on; hold on;

plot(ax_ins4_2, t, results_dempc.Pae / 1000, 'b-', 'LineWidth', 1.5);

plot(ax_ins4_2, t, results_gwo.Pae / 1000, 'r--', 'LineWidth', 1.5);

xlim(ax_ins4_2, xlim_pae);

ylim(ax_ins4_2, [ymin_pae - margin_pae, ymax_pae + margin_pae]);

%title(ax_ins4_2, 'Zoom: 0.19 - 0.23 s', 'FontSize', 8);

% FC Power

ax4_3 = subplot(2, 2, 3);

plot(t, results_dempc.Ppe / 1000, 'b-', 'LineWidth', 1.5); hold on;

plot(t, results_gwo.Ppe / 1000, 'r--', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('P_{pe} (kW)');

% lgd = legend('DMPC', 'Proposed GWO-DMPC', 'P_{max}', 'Location', 'SouthWest');

% lgd.FontSize = 5;

% lgd.ItemTokenSize = [10, 8];

% legend boxoff;

lgd = legend(ax4_3, 'DMPC', 'Proposed GWO-DMPC');

set(lgd, 'Position', [0.15, 0.20, 0.1, 0.03]); % [left, bottom, width, height]

lgd.FontSize = 5;

lgd.ItemTokenSize = [8, 8];

%title('Fuel Cell Power');

% text(ax4_3, 0.5, -0.15, '(c). Fuel Cell Power', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

text(ax4_3, 0.5, -0.20, '(c)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

grid on; xlim([0, 0.8]);

% --- Zoom on P_fc ---

xlim_pfc = [0.58, 0.65];

idx_pfc = (t >= xlim_pfc(1) & t <= xlim_pfc(2));

ymin_p = min([min(results_dempc.Ppe(idx_pfc)), min(results_gwo.Ppe(idx_pfc))]) / 1000;

ymax_p = max([max(results_dempc.Ppe(idx_pfc)), max(results_gwo.Ppe(idx_pfc))]) / 1000;

margin_p = max(0.5, (ymax_p - ymin_p) * 0.1);

rectangle(ax4_3, 'Position', [xlim_pfc(1), ymin_p-margin_p, xlim_pfc(2)-xlim_pfc(1), (ymax_p+margin_p)-(ymin_p-margin_p)], 'EdgeColor', 'k', 'LineStyle', '-.');

pos_pfc = get(ax4_3, 'Position');

ax_ins4_3 = axes('Position', [pos_pfc(1)+pos_pfc(3)*0.1, pos_pfc(2)+pos_pfc(4)*0.55, pos_pfc(3)*0.4, pos_pfc(4)*0.4]);

box on; grid on; hold on;

plot(ax_ins4_3, t, results_dempc.Ppe / 1000, 'b-', 'LineWidth', 1.5);

plot(ax_ins4_3, t, results_gwo.Ppe / 1000, 'r--', 'LineWidth', 1.5);

xlim(ax_ins4_3, xlim_pfc);

ylim(ax_ins4_3, [ymin_p - margin_p, ymax_p + margin_p]);

%title(ax_ins4_3, 'Zoom: 0.58 - 0.65 s', 'FontSize', 8);

% Battery Power

ax4_4 = subplot(2, 2, 4);

plot(t, results_dempc.Pbat / 1000, 'b-', 'LineWidth', 1.5); hold on;

plot(t, results_gwo.Pbat / 1000, 'r--', 'LineWidth', 1.5);

xlabel('Time (s)');

ylabel('P_{bat} (kW)');

% lgd = legend('DMPC', 'Proposed GWO-DMPC', 'P_{max}', 'Location', 'SouthWest');

% lgd.FontSize = 5;

% lgd.ItemTokenSize = [10, 8];

% legend boxoff;

lgd4 = legend(ax4_4, 'DMPC', 'Proposed GWO-DMPC');

set(lgd4, 'Position', [0.59, 0.2, 0.1, 0.03]); % [left, bottom, width, height]

lgd4.FontSize = 5;

lgd4.ItemTokenSize = [8, 8];

%title('Battery Power');

% text(ax4_4, 0.5, -0.15, '(d). Battery Power', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

text(ax4_4, 0.5, -0.20, '(d)', 'Units', 'normalized', 'HorizontalAlignment', 'center', 'FontWeight', 'bold');

grid on; xlim([0, 0.8]);

% --- Zoom on P_bat ---

xlim_pbat = [0.58, 0.65];

idx_pbat = (t >= xlim_pbat(1) & t <= xlim_pbat(2));

ymin_p = min([min(results_dempc.Pbat(idx_pbat)), min(results_gwo.Pbat(idx_pbat))]) / 1000;

ymax_p = max([max(results_dempc.Pbat(idx_pbat)), max(results_gwo.Pbat(idx_pbat))]) / 1000;

margin_p = max(0.5, (ymax_p - ymin_p) * 0.1);

rectangle(ax4_4, 'Position', [xlim_pbat(1), ymin_p-margin_p, xlim_pbat(2)-xlim_pbat(1), (ymax_p+margin_p)-(ymin_p-margin_p)], 'EdgeColor', 'k', 'LineStyle', '-.');

pos_pbat = get(ax4_4, 'Position');

ax_ins4_4 = axes('Position', [pos_pbat(1)+pos_pbat(3)*0.1, pos_pbat(2)+pos_pbat(4)*0.55, pos_pbat(3)*0.4, pos_pbat(4)*0.4]);

box on; grid on; hold on;

plot(ax_ins4_4, t, results_dempc.Pbat / 1000, 'b-', 'LineWidth', 1.5);

plot(ax_ins4_4, t, results_gwo.Pbat / 1000, 'r--', 'LineWidth', 1.5);

xlim(ax_ins4_4, xlim_pbat);

ylim(ax_ins4_4, [ymin_p - margin_p, ymax_p + margin_p]);

%title(ax_ins4_4, 'Zoom: 0.58 - 0.65 s', 'FontSize', 8);

% =========================================================================

% FIGURE 5: Battery SOC and Hydrogen Tank Pressure

% =========================================================================

f5 = figure('Name', 'Figure 5: Energy Storage States', 'Position', [300, 200, 800, 500], 'Color', 'w');

% Battery SOC

subplot(2, 1, 1);

plot(t, results_dempc.SOC * 100, 'b-', 'LineWidth', 2); hold on;

plot(t, results_gwo.SOC * 100, 'r--', 'LineWidth', 2);

yline(90, 'k--', 'Upper Limit (90%)', 'LabelHorizontalAlignment', 'left', 'LineWidth', 1.2);

yline(20, 'k--', 'Lower Limit (20%)', 'LabelHorizontalAlignment', 'left', 'LineWidth', 1.2);

xline(stages, 'k:');

ylabel('SOC (%)');

%title('Battery State of Charge');

legend('DMPC', 'Proposed GWO-DMPC', 'Location', 'best');

grid on; xlim([0, 0.8]);

% Hydrogen Tank Pressure

subplot(2, 1, 2);

plot(t, results_dempc.Ptank / 1e5, 'b-', 'LineWidth', 2); hold on;

plot(t, results_gwo.Ptank / 1e5, 'r--', 'LineWidth', 2);

xline(stages, 'k:');

xlabel('Time (s)');

ylabel('Pressure (bar)');

%title('Hydrogen Tank Pressure');

grid on; xlim([0, 0.8]);

% =========================================================================

% Export Figures

% =========================================================================

% fprintf('Saving figures to PDF (500 DPI)...\n');

% exportgraphics(f1, 'Figure_1_Scenario_EMS.jpeg', 'ContentType', 'vector', 'Resolution', 500);

% exportgraphics(f2, 'Figure_2_DC_Bus_Voltage.jpeg', 'ContentType', 'vector', 'Resolution', 500);

% exportgraphics(f3, 'Figure_3_Duty_Cycles.jpeg', 'ContentType', 'vector', 'Resolution', 500);

% exportgraphics(f4, 'Figure_4_Power_Dispatch.jpeg', 'ContentType', 'vector', 'Resolution', 500);

% exportgraphics(f5, 'Figure_5_Energy_Storage.jpeg', 'ContentType', 'vector', 'Resolution', 500);

% =========================================================================

% Console Notification

% =========================================================================

% fprintf('All visualizations generated successfully.\n');

% fprintf(' - Figure 1: Scenario & EMS\n');

% fprintf(' - Figure 2: DC Bus Voltage Regulation\n');

% fprintf(' - Figure 3: Duty Cycle Trajectories\n');

% fprintf(' - Figure 4: Power Dispatch\n');

% fprintf(' - Figure 5: Energy Storage States\n');